AIR QUALITY INDEX REVISITED

FROM A COMPOSITIONAL APPROACH

Eusebi Jarauta-Bragulat1, Carme Hervada-Sala2 and Juan José Egozcue3

1 Dept. of Applied Mathematics III. Technical University of Catalonia (UPC). eusebi.jarauta@upc.edu

2 Dept. of Physics and Nuclear Engineering. Technical University of Catalonia (UPC). carme.hervada@upc.edu

3Dept. of Applied Mathematics III. Technical University of Catalonia (UPC). juan.jose.egozcue@upc.edu

Abstract.

Air quality has become a social concern due to its consequences on population's health. A large amount of air quality indices (AQIs) have been defined. Computing AQI values requires measuring air pollutant concentrations and a model providing the function used to convert from air pollutant concentration to AQI. The air quality indices values are divided into ranges and for each range a descriptor and a colour code are assigned. Standardized public health advisories are associated with each AQI range. The presence of contaminants in air is expressed by their concentration, usually in ppm or µg/m3. Daily and monthly averages of concentrations are carried out using the arithmetic mean of the corresponding measured values for each element. These average values are used in the model to assign the AQI values for each pollutant. The final air quality report usually contains: the maximum AQI of different air pollutants, the average of the two greatest ones and the arithmetic mean of all AQIs, the overall AQI. Concentrations of pollutants are compositional data, that is, parts of a whole; therefore they only provide ratio information between concentrations of pollutants (parts). Operations involved in the computation of overall AQI are not admissible operations in the framework of compositional data analysis, as they destroy the original ratio information. Consequently, this methodology should be reviewed for such calculations, taking into account the operations and principles of compositional data analysis.

From the point of view of compositional data analysis, daily/monthly averages should be computed using the centre of the sample, i.e., the closed geometric mean of the concentrations. However, we pay attention to the average of the pollutants AQIs. We propose to use a balance log-contrast, which is approximately equal to the geometric mean, of concentrations expressed in µg/m3 to define a new AQI indicator. This balance has a clear interpretation as related to an orthogonal projection in the geometry of the simplex. Furthermore, the geometric mean of concentrations, allow us to obtain a new and simple scale expressing a qualitative air quality index. As an illustrative example, statistical analysis of atmospheric pollution data series (2004-2013) of the city of Madrid (Spain) has been carried out.

Keywords. Air pollution, Air Quality Index, Compositional data analysis, Logcontrast.

1 	Introduction

In recent years, air quality has become a social concern due to its consequences on population's health. Nowadays, there is a clear society demand to live with reasonable environmental quality, and one of the most important environmental items is air quality. In big cities, where there is large population concentration, a reasonable good air quality is required. On top of this, there is a call for timely and reliable information on air quality, which implies a good data management and measures to protect health, if needed.

In this context, air quality must be defined, measured and expressed in an adequate and comprehensible way. Authorities must undertake a proper air quality management and population should be able to understand measures taken in order to keep air quality under thresholds. To quantify air quality, some indices (AQIs) have been defined, all related to pollutants concentration in air. The most used pollutants are ozone (O3), carbon monoxide (CO), sulphur dioxide (SO2), nitrogen dioxide (NO2) and suspended particles classified by their maximum diameter (PM25, PM10).

We have analysed the most common AQIs: AQI defined by the U.S. Environmental Protection Agency (http://www.epa.gov/), the CAQI defined in the European project CITEAIR (van den Elshout et al. (2008)) and the revised AQI proposed by Plaia et al. (2013). Plaia and Ruggieri (2011) review air quality indices and Bishoi et al. (2009) do a comparative study of them. Some authors have incorporated geostatistical concepts and methods, like Russo and Soares (2014). Computing AQI values requires measuring air pollutant concentrations and a model providing the function used to convert from air pollutant concentration to AQI values. Computing AQI values is done using a piecewise linear function, which transforms concentrations into AQI normalized values. Breakpoints are chosen to fit observed and subjectively evaluated health impacts. The AQI calculation is

|  | (1.1) |
| --- | --- |

where AQI(P) is AQI value for pollutant P; C(P) is the average concentration for pollutant P; BPHI is the breakpoint that is greater than or equal to C(P); BPLO is the breakpoint that is less than or equal to C(P); AQIHI is the AQI value corresponding to BPHI and AQILO is the AQI value corresponding to BPLO. Such breakpoints vary by pollutant, and are different in different countries. Air quality index values are divided into ranges, and each range is assigned a descriptor and a colour code (Table 1.1, Figure 1.1). Standardized public health alert protocols are associated with each AQI range. The presence of contaminants in air is expressed by their concentration, usually in ppm or µg/m3. Daily and monthly averages of concentrations are carried out using the arithmetic mean of the corresponding measured values for each element and over the corresponding period of time. These average values are used in the model to assign the AQI values for each pollutant. The final air quality report usually contains: the maximum AQI of different air pollutants, the average of the two greatest ones and the arithmetic mean (average) of all AQIs, the overall AQI.

Concentrations of air pollutants are compositional data. They describe the parts in which the whole amount of air is decomposed. This means that the information provided by compositional data comes from the ratios between different parts. The analysis of compositional data must be done according to some principles (Aitchison (1986)). For instance, scale invariance requires that a change of units in concentrations does not change the result of the analysis; or, subcompositional coherence preconizes that analysis performed on different subcompositions give the same information for the common elements or parts. Violation of some of these principles may lead to inconsistent results. The very first reference about such problems was pointed out by K. Pearson (1897), one of the founders of modern statistics, who pointed out the spurious correlation phenomenon.

Figure 1.1: Piecewise linear function assigning AQI values to concentration of pollutant.

Table1.1  AQI breakpoints and corresponding colour codes.

|  | AQI |  |  |
| --- | --- | --- | --- |
|  | Low | High |  |
| Good | 0 | 50 |  |
| Moderate | 51 | 100 |  |
| Unhealthy 1 | 101 | 150 |  |
| Unhealthy 2 | 151 | 200 |  |
| Very unhealthy | 201 | 300 |  |
| Hazardous | 301 | 400 |  |
| Very hazardous | 401 | 500 |  |

In the eighties J. Aitchison (1986) put forward the so called log-ratio approach, defining suitable principles and techniques to deal with compositional data. From then on, there have been significant advances in the formal aspects of the analysis that allow greater systematization of his methods. Summing up, statistical analysis of compositional data can be stated as a three step process: (1) transforming data in a log-ratio coordinates; (2) statistical analysis of those coordinates as real variables; (3) interpretation of results on the coordinates or back-transforming them into compositional data. However, the use of this technique is not as widespread as it deserves (Egozcue and Pawlowsky-Glahn (2011a)). The main goal of this paper is to contribute to improve measurement, interpretation and management of air quality using compositional data techniques. The purpose of this study is essentially descriptive; further research would be necessary to get a complete, coherent and comprehensive air quality theory. For instance, evolution in time of air pollutants can be modelled using differential equations in the simplex (Egozcue et al. (2011)).

2 	Methodology

A data set granted by Ayuntamiento de Madrid (Spain) is available. The monthly mean concentration of O3, PM2.5, PM10, CO, SO2, NO2 is reported from 2004 to 2013, constituting a sample of  n = 117 monthly data points.

The first step in the analysis was to compare averages of pollutant concentrations and averages of corresponding AQIs. The NO2 data values are removed for this analysis, because this pollutant has not short term for AQI computation. As the functions relating concentrations to AQIs are piecewise linear functions of the pollutant concentrations (Eq (1.1)), it would be expected a good correlation between arithmetic average of pollutant concentrations and arithmetic average of AQIs, which is the overall AQI. Figure 2.1(a) shows time evolution of the arithmetic mean of pollutant concentration and the arithmetic mean of pollutant AQIs. Figure 2.2(a) shows the scatterplot of pollutant concentration arithmetic average versus AQI arithmetic average; the correlation coefficient for these two variables is 0.579, which is a value smaller than expected. Moreover the shape of the two curves in Figure 2.1(a) is clearly different. Figure 2.1(b) shows time evolution of the geometric mean of pollutant concentration and the geometric mean of pollutant AQIs. Figure 2.2(b) shows the scatterplot of pollutant concentration geometric mean versus AQI geometric mean; the correlation coefficient for these two variables is 0.999, and the shape of the two curves fits very well. The analysis illustrated in Figures 2.1 and 2.2 suggests that geometric mean is a better centrality estimator than arithmetic mean when dealing with concentrations. This confirms well known facts in compositional data analysis (Aitchison (1986), Aitchison and Egozcue (2005)).

(a)

(b)

Figure 2.1: (a) pollutant concentration arithmetic mean (green) and AQI arithmetic mean (purple) as a function on time; (b) pollutant concentration geometric mean (green) and AQI geometric mean (purple) as a function of time (data set Madrid 2004-2013).

(a)

(b)

Figure 2.2: (a) scatterplot of pollutant concentration arithmetic mean versus AQI arithmetic mean; (b) scatterplot of pollutant concentration geometric mean versus AQI geometric mean.

(Data set Madrid 2004-2013).

Table 2.1  Correlation matrices of air pollutants in Madrid showing the effect of spurious correlation.  (a) when concentrations of pollutants are expressed in ppb of mass; (b) when concentrations of pollutants are expressed of proportions in mass in the subcomposition

O3, PM2.5, PM10, CO, SO2

| (a) | O3 | PM2.5 | PM10 | CO | SO2 |
| --- | --- | --- | --- | --- | --- |
| O3 | 1 |  |  |  |  |
| PM2.5 | -0.361 | 1 |  |  |  |
| PM10 | -0.330 | 0.179 | 1 |  |  |
| CO | -0.714 | 0.344 | 0.583 | 1 |  |
| SO2 | -0.641 | 0.060 | 0.605 | 0.814 | 1 |

| (b) | O3 | PM2.5 | PM10 | CO | SO2 |
| --- | --- | --- | --- | --- | --- |
| O3 | 1 |  |  |  |  |
| PM2.5 | 0.443 | 1 |  |  |  |
| PM10 | 0.353 | 0.073 | 1 |  |  |
| CO | -0.924 | -0.569 | -0.624 | 1 |  |
| SO2 | -0.164 | -0.379 | 0.486 | 0.003 | 1 |

Concentrations of air pollutants constitute a compositional vector, a composition for short. Compositions represent parts of whole, in this case total micrograms in a cubic meter of air. The information conveyed by a composition is summarized in the ratios between different parts. Therefore, when multiplying a composition by a constant, information does not change and the two vectors are equivalent as compositions. For instance, to express air pollutants in µg/m3 is equivalent to express them in other units, e.g. in g/m3. Another characteristic of compositions is the subcompositional coherence. Analysis made in a subset of parts of the original composition, should not produce different results referred to common parts. An example of violation of this principle is shown in Table 2.1 which shows two correlation matrices of air pollution data set computed on the whole composition and computed on a subcomposition. See for instance that correlation CO-PM10 in case (a) is 0.583, meaning that when CO increases PM10 also increases; in case (b) the correlation is 0.624, meaning that when CO increases PM10 decreases, an incoherent interpretation.

A proper compositional data analysis should be performed through functions of concentrations which are invariant under change of units or multiplication by constants (Egozcue 2009). On the other hand, the scale of ratios is commonly treated taking logarithms, as proposed in Aitchison (1986). Compositional data can be properly analysed and represented using the expressions called log-contrasts. For a composition  x = (x1, x2, …, xn), a log-contrast (Egozcue and Pawlowsky-Glahn, (2011b)) is a linear combination of logarithms of parts where the sum of the coefficients is zero:

|  | (2.1) |
| --- | --- |

which is a scale invariant function. Some of the ai should be positive, call them bi, and the rest negative, call them ci, or null, call them di. So, Eq.(2.1) can be written as:

|  | (2.2) |
| --- | --- |

A very important and interesting property of any log-contrast is that, if a change of units is applied to one or more components of the numerator or denominator, then the only change in the log-contrast is adding a known constant. For instance,

|  | (2.3) |
| --- | --- |

When working with air pollutant concentrations, the original composition is made of concentration of pollutants and a fill-up value, consisting of other air components. If they are expressed in ppb, they add up to 109, as it was done in Table 2.1(a). Calling the fill-up value  xn, the log-contrast

|  | (2.4) |
| --- | --- |

is an index of air quality, denoted bAQ, as it is a balance between non-polluted air and considered air  pollutants. In Eq.(2.4), gn-1(·) is the geometric mean of n1 first components, usually 6 pollutants. Note that bAQ is dimensionless, that is, it does not depend on the units in which concentrations are expressed. However, both xn and gn1(x) have the units of concentrations, e.g.,  g/m3 , ppm or ppb. In practice, log(xn) is almost constant across the sample, and therefore, bAQ can be approached as  log(gn1(x)) up to an additive constant. The quantity log(gn1(x)) is not dimensionless and only is useful to compare different pollutant concentration values, as the additive constants cancels out. Fig. 2.3 shows how bAQ  and log(gn1(x)) differs approximately in a constant term.

From the point of view of compositional data analysis, the use of log-contrasts is much more suitable and coherent than using concentrations or even log-concentrations. On top of this, we think that air quality index can be redefined using the log-contrast bAQ or its approximation by geometric mean of pollutant concentrations. The main advantage of this approach is that bAQ is dimensionless and does not depend on the units of concentrations. Moreover, changing units of concentrations, modify the value of log(gn1(x)) adding a constant.

Figure 2.3: log-contrast using residual component (blue) and using a constant (red),

(Data set Madrid 2004-2013).

3 	Proposal of a new Air Quality Index

The common practice in air quality evaluation consists of defining AQI indexes for each pollutant. These indexes are then combined to obtain global AQI values; for instance, the arithmetic average of all pollutant-AQI, the maximum pollutant-AQI and the average of the two maximum pollutant-AQI values. Some authors (Plaia et al. (2013), Bruno and Cocchi (2002)) have suggested defining the air quality index as a certain global result, taking into account pollutant concentrations and not through the pollutant-AQI. In order to define a new global air quality index directly based on the air pollutants concentration, the log-contrast bAQ is a natural choice from the compositional point of view. However, the fill-up value on the composition is almost never reported and so bAQ is not easily computed. In its place, we propose to use the geometric mean of pollutants concentration   gn1(x). This new air quality index, denoted AQI*, is computed as

|  | (3.1) |
| --- | --- |

where the coefficient k* depends on the units of concentrations. Once units of concentration are fixed (e.g. g/m3), the value of the constant k* depends on the desired scale of AQI indexes. The common practice assigns values of overall AQI to the so called breakpoints of pollutants concentration (see Table 3.1). For instance, when the concentrations of pollutants are those in the second row, AQI of 100 is assigned. However, the individual AQI value of 100 is also assigned if for instance PM10 is 154.0, and the rest of concentrations are under the corresponding breakpoint.

Table 3.1: columns from O3 to NO2, concentration pollutants breakpoints (µg/m3).

Traditional AQI values corresponding to breakpoints. GM: geometric mean of pollutant concentrations excluding NO2. AQI*, scaled from 0 to 100.

| O3 | PM2.5 | PM10 | CO | SO2 | NO2 | AQI | GM | AQI* |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 129.9 | 15.4 | 54.0 | 5210.4 | 92.0 | 0.0 | 50 | 139.0 | 6 |  |
| 170.5 | 40.4 | 154.0 | 11131.4 | 389.8 | 0.0 | 100 | 340.9 | 15 |  |
| 211.1 | 65.4 | 254.0 | 14684.0 | 606.3 | 0.0 | 150 | 499.9 | 22 |  |
| 251.7 | 150.4 | 354.0 | 18236.6 | 822.8 | 1044.6 | 200 | 725.6 | 32 |  |
| 759.2 | 250.4 | 424.0 | 35999.4 | 1634.9 | 1992.8 | 300 | 1365.3 | 60 |  |
| 1023.1 | 350.4 | 504.0 | 47841.4 | 2176.2 | 2635.7 | 400 | 1798.4 | 79 |  |
| 1226.1 | 500.4 | 604.0 | 59683.3 | 2717.6 | 3278.5 | 500 | 2268.7 | 100 |  |

Figure 3.1: AQI* values vs AQI values at the traditional breakpoints (blue dots). Fitted line (red).

Figure 3.2: AQI values (red) and AQI* values (blue) in time. (Data set Madrid 2004-2013).

We accept that the AQI scale given by the breakpoints in Table 3.1 is a reasonable and interpretable scale. On the other hand, it seems also reasonable that the new index to have a range from 0 to 100. A comparison between breakpoints AQI and the geometric mean of pollutant concentrations (excluding NO2) is shown in Figure 3.1. As linear fitting of AQI* and traditional AQI is good, we accept they share the same scale. In table 3.1 we observe that GM (the geometric mean of pollutant concentrations excluding NO2) ranges from 0 up to 2268.7. Accordingly, the normalization from 0 to 100 is computed as

|  | (3.2) |
| --- | --- |

where concentrations in the vector are expressed in µg/m3. Note that NO2 was not used for assessing the scale of AQI*, but the value 2268.7 including NO2 in the geometric mean is an almost incredible air pollution situation. Equation (3.1) is maintained as the definition of AQI* although the geometric mean includes NO2. Figure 3.2 shows a comparison between AQI and AQI* values computed for the reference data set (Madrid, 2004-2013); a secondary scale must be used, as the corresponding scales are different although they are almost linear related (Figure 3.1). Figure 3.2 also shows that air pollution in Madrid was in general very low (AQI roughly ranges from 10 to 30 and AQI* from 0.5 to 2.5). We would recommend to give AQI* in percentage, so that the AQI* range for this data is said about 1%  2% polluted.

Conclusions

Air quality is measured based on the concentrations of certain pollutants and concentration of suspended particles classified by size; those concentrations are compositional data and, consequently, they are properly analysed using log-ratio approaches. These analyses are based on log-contrasts, as they are scale invariant. The log-contrast between concentration of non-pollutant components over pollutant components is taken as a proper dimensionless index of air pollution. A good approximation of this index is the logarithm of the geometric mean of concentrations of air pollutants. The scale of this geometric mean is comparable to the traditional AQI scale. A new air quality index ranging from 0 to 100 is proposed as AQI*(x) = k*gn-1(x), where k* is a constant depending on the pollutants concentration units and gn-1(·) is the geometric mean of air pollutants concentrations.

Acknowledgments

This work has been supported by the Ministerio de Economía y Competitividad of Spain under projects ENE2012-36871-C02-01 and METRICS Ref. MTM2012-33236.

Authors also are thankful to Excmo. Ayuntamiento de Madrid for granting air pollution data.

References

[1] van den Elshout, S., Léger, K. and Nussio, F. (2008), Comparing urban air quality in Europe in real time. A review of existing air quality indices and the proposal of a common alternative. DCMR, Environmental International, 34: 720-726.

[2] Plaia, A., Di Salvo, F., Ruggieri, M. and Agró, G. (2013), A multisite-multipollutant air quality index. Atmospheric Environment, 70: 387-391.

[3] Plaia, A. and Ruggieri, M. (2011), Air quality indices: a review. Rev Environ Sci Biotechnol, vol. 10: 165-179.

[4] Bishoi, B., Prakash, A. and Jain, V.K. (2009), A comparative study of air quality index based on factor analysis and US-EPA methods for an urban environment. Aerosol and air quality research, Vol. 9, No. 1: 1-17.

[5] Russo, A. and Soares, A. (2014), Hybrid model for urban air pollution forecasting: a stochastic spatio-temporal approach. Mathematical Geosciences, 46, 75-93.

[6] Aitchison, J. (1986), The Statistical Analysis of Compositional Data. Monographs on Statistics and Applied Probability. Chapman & Hall Ltd. London (UK). (Reprinted in 2003 with additional material by The Blackburn Press), 416 pp.

[7] Pearson, K. (1897), Mathematical contributions to the theory of evolution. On a form of spurious correlation which may arise when indices are used in the measurement of organs. Proceedings of the Royal Society of London, LX, 489-502.

[8] Egozcue, J.J. and Pawlowsky-Glahn, V. (2011a), Basic concepts and procedures. In Compositional Data Analysis. Theory and Applications, Chap 2, edited by Vera Pawlowsky-Glahn and Antonella Buccianti: 12-28. John Wiley & Sons.

[9] Egozcue, J.J., Jarauta-Bragulat, E. and Díaz-Barrero, J.L. (2011), Calculus of simplex-valued functions. In Compositional Data Analysis. Theory and Applications, Chap. 12, edited by Vera Pawlowsky-Glahn and Antonella Buccianti: 158-175. John Wiley & Sons.

[10] Aitchison, J. and Egozcue, J.J. (2005), Compositional data analysis: where are we and where should we be heading? Mathematical Geology, 37(7): 829-850.

[11] Egozcue, J.J. (2009), Reply to “On the Harker variation diagrams; …” by J.A. Cortés.  Mathematical Geology, 41(7): 829-834.

[12] Egozcue, J.J. and Pawlowsky-Glahn, V. (2011b), Análisis composicional de datos en ciencias geoambientales. Boletín Geológico y Minero, 122(4): 439-452.

[13] Bruno, F. and Cocchi, D. (2002), A unified strategy for building simple air quality indices. Environmetrics; 13: 243-261.