GEOSTATISTICAL STUDIES OF SPACE-TIME VARIATION IN UNDERGROUND WATER QUALITY PARAMETERS IN KLODZKO WATER INTAKE AREA (SW PART OF POLAND)

Barbara Namysłowska-Wilczyńska1

1 Institute of Geotechnics and Hydrotechnics Wroclaw University of Technology,

Wybrzeże Wyspiańskiego 27, 50 – 370 Wrocław, Poland, Barbara.Namyslowska-Wilczynska@pwr.edu.pl

Abstract. This paper presents selected results of research connected with the development of a (3D) geostatistical hydrogeochemical model of the Kłodzko Drainage Basin, dedicated to the spatial and time variation in the topographical, hydrogeological and quality parameters of underground water in the Kłodzko water intake area (SW part of Poland) (2013) [1], (2012) [3], (2013) [4], (2013) [5]. The research covers the period 1977 - 2012. Spatial analyses of the variation in different quality parameters, i.e. Fe [gFe/m3], Mn [gMn/m3], ammonium ion [gNH4+/m3], oxidation capacity [gO2/ m3], nitrite anion NO3- [gNO3/m3] and phosphate anion PO4-3 [gPO4-3/m3] contents values, total organic carbon (TOC) content C [5gC/m3], pH redox potential and temperature C [degrees], were carried out on the basis of the chemical determinations of water samples (2012) [3], (2013) [4], (2013) [5]. Spatial and time variation in the parameters was analyzed on the basis of archival data (1977-1999) for 22 (pump and siphon) wells, later  data obtained (November 2011) from tests of water taken from 14 wells and the latest data (January 2012) acquired from three piezometers.

Thematic databases, containing original data on coordinates X, Y (latitude and longitude) and Z (terrain elevation and time – years) and on regionalized variables, i.e. the underground water quality parameters in the Kłodzko water intake area determined for different analytical configurations (22 wells, 14 wells, 14 wells + 3 piezometers), were created (2012) [3]. The input for the studies were the chemical determinations of the quality parameters of underground water samples taken from the wells in the water intake area in different periods of time. Both archival data (acquired in the years 1977 - 1999, 1977 - 2011) and the latest data (collected in November 2011 and in January 2012) were analyzed. These data were subjected to spatial analyses using statistical and geostatistical methods 2012 [3], (2013) [4], 2013 [5]. The directional semivariogram function and the ordinary (block) kriging procedure were used to build the 3D geostatistical model. The geostatistical parameters of the theoretical models of directional variograms of the studied water quality parameters, calculated along the well depth axis (taking into account the terrain elevation) and the time interval, were used in the ordinary (block) kriging estimation.

Generally, the behaviour of the underground water quality parameters has been found to vary in space and time. Thanks to the multidirectional spatial analyses of the variation in the quality parameters in the Kłodzko underground water intake area some regularities in the variation in water quality have been identified. The obtained results of interpolation and estimation, allowed to determine the levels of elevated values ​​of studied underground water quality parameters (2012) [3], (2013) [5]. Frequently maximum average values ​​of Z* parameters were found at levels of 50 or 60, rarely 70, related to the height of the terrain interval from 290.55 - 292.36 m above sea level, from the SW part of the area in question intakes (less of N, NW and NE parts).

Geostatistical model (3D) of hydrogeological and groundwater quality variability parameters has allowed to obtain precise characteristics of these parameters throughout the area under consideration water intake area over the years 1977 – 2012 (2012) [3], (2013) [5].

Keywords. Underground Waters, Hydrogeological Parameters, Quality Parameters, Space-Time Variation, Geostatistics, Directional Semivariogram, Ordinary Kriging, Hydrogeochemical Model (3D).

1 	Extended abstract

A 3D hydrogeochemical model of the variation in hydrogeological and water quality parameters in the Kłodzko Quaternary ground water intake area near Kłodzko (the Kłodzko Neisse and Bystrzyca Dusznicka drainage area), which includes the terrain topography, is presented. Geostatistical methods, i.e. the directional semivariogram function and estimation techniques: ordinary (block) kriging and quick (inverse squared distance) interpolation, were used to build the model (2012) [3], (2013) [5].

The subject of the spatial analyses was the variation evaluation (in the horizontal and vertical extents and in time) in the values of the hydrogeological and water quality parameters in the area of ground water intakes near the town of Kłodzko (2012) [3], (2013) [4], (2013) [5].

The inhabitants of Kłodzko are supplied with water by a central water main drawing off water from underground intakes in Quaternary formations [1], [2], [4]. Water is drawn off (siphoned or pumped) via dug and drilled wells located on both sides of the Kłodzko Neisse river. Generally, the depth of the wells ranges from 9.5 to 38.0 m. The Quaternary formations are deposited directly on crystalline metamorphic rocks. These are Pleistocene deposits dating back to the Cracow and Central Poland glaciation period, consisting of fluvioglacial accumulation sands and gravels and river accumulation gravels and sands forming well-preserved sand-gravel terraces, boulder clays, varved clays and loess-like clays. The younger Quaternary (Holocen) formations, i.e. silts, sands and gravels, are deposited in the river valleys and in streams.

Boreholes have shown that the oldest formations are Old Palaeozoic greenstones or their weathering waste, underlaying the Quaternary sand-gravel deposits. Greenstones were found in the deepest boreholes, maximally to 38 m below the terrain surface, but generally their roof is deposited at 20-36 m, coming closer (6-10 m) to the surface towards the edges of the formed through. This means that the roof forms the uneven below-Quaternary surface of the Kłodzko Neisse valley.

In the N part of the water-bearing area the permeable layer floor is situated at a depth of 18.5-21.5 m b.g.l. The layer is made up of pebbles, gravels and sands. Whereas in the S part of this area the poorly permeable layer roof (Old Palaeozoic greenstone weathering waste) lies at a depth of 34.5-38 m b.g.l. The aquifer is made up of sands with an admixture of river accumulation gravels and  pebbles. The hydrogeochemical characteristics of environments of river valleys was presented in the work [2].

The input for the geostatistical studies were the results of chemical analyses of underground water samples (including water quality parameters) carried out for the wells in the water intake area in different periods of time, and heavy metal content determinations for the town of Kłodzko. The water quality data included: archival data acquired in the 1970s, data acquired in the periods 1977-1999 and 1977-2011 and recent data collected on 15.11.2011 and 23.01.2012.

Various thematic databases, containing values of coordinates X, Y (latitude and longitude) and Z (terrain elevation, and time – years) and regionalized variables, i.e. the topographic parameter and hydrogeological and underground water quality parameters, were created. Spatial and space-time analyses were carried out for several analytical variants, taking into account data from 22 wells, 14 wells, 14 wells + 3 piezometers and 3 piezometers in the Kłodzko water intake area, using the computing programmes included in  the ISATIS software package.

First data from 22 (pump and siphon) wells built in the years 1954-1998  in the Kłodzko water intake area were analyzed. 14 wells plus a collector siphon well (15.11.2011) were selected for further analyses aimed at determining the current underground water condition. At a later date (22-23.01.2012) 3 new piezometers were made in the water intake area whereby the existing databases of boreholes (wells) and so the set of chemical determination results could be expanded.

The variation in the values of the topographic parameter (terrain elevation), hydrogeological parameters (water abstraction level Z with and without the landform features and the water table depth taken into account) and various water quality parameters (in space and time) was studied on the basis of the data coming from 22, 14 and 14 wells and 3 piezometers. Also the variation in the heavy  metal content in water coming from the 3 piezometers was investigated. Initially, data coming from chemical determinations carried out for the 22 wells with terrain elevations in a range of 287.22-297.70 m a.s.l. were analyzed. Spatial analyses of the variation in:

- the topographic parameter (terrain elevation (m a.s.l),

- hydrogeological parameters: water abstraction level Z (m a.s.l) – the dynamic water table  and the depth of occurrence of the underground water table (m b.g.l.),

- water quality parameters: Fe content [gFe/m3], Mn content [gMn/m3], NH4+ (ammonium ion) content [gNH4+/m3] and oxidation capacity [gO2/m3],

within the underground water intake area were carried out.

Then the data on the water samples taken from only the 14 wells, and also from the collector siphon well located in the Kłodzko water intake area, were studied. Spatial analyses of the following 8 underground water quality parameters were carried out: Fe content [gFe/m3], Mn content [gMn/m3], ammonium ion content [gNH4+/m3], NO3- (nitrate ion) content [gNO3/m3], PO4-3 (phosphate ion) content [gPO4-3/m3], pH redox potential and temperature C [degrees] (2012) [3].

Then chemical analyses of underground water samples taken from the three piezometers (P1, P2, P3) with a depth of 9-10 m were performed whereby the set of the existing test results was expanded (2012) [3].  In total 26 water quality and chemical parameters, such as:

- pH reaction,

- electrical conductivity (25C) – the normalized value – 2500 [µS/cm],

- Na content [g/m3], K content [g/m3] and Ca content – unnormalized values, Mg content – the normalized value – 30-125 [g/m3], Mn content < 0.05 [g/m3], Cl (chloride) content – the normalized value – 250 [g/m3], F (fluoride) content – the normalized value – 1.50 [g/m3], SO4-2 (sulphate anion) content – the normalized value – 250 [g/m3],

- total Fe content < 0.20 [g/m3], Zn content [g/m3], Cu content – 2.00 [g/m3], total Cr content – 0.05 [g/m3], Cd content – 0.005 [g/m3] and Pb content – 0.025 [g/m3],

- hydrogencarbonate content [mg/dm3] – an unnormalized value,

- carbonate content [mg/dm3] – an unnormalized value,

- hydroxide content [mg/dm3] – an unnormalized value,

- total alkalinity [mmol/dm3] and alkalinity towards phenolphthalein [mmol/dm3] – unnormalized values,

- total hardness – 60-500 [gCaCO3/m3], [mmol/dm3], carbonate hardness and non-carbonate hardness [gCaCO3/m3], [mmol/dm3] – unnormalized values,

- solute content [g/m3] – an unnormalized value,

- permanganate index (oxidation capacity) – the normalized value – 5[g/m3].

The data coming from both the 3 piezometers and the 17 holes (14 wells + 3 piezometers) (water determinations for the 3 piezometers located in the intake area were added to the database for the 14 wells) were subjected to analysis (2012) [3].

The basic statistics were evaluated and histograms of water quality parameters distribution were calculated (2012) [3], (2013) [4]. Moreover, the degree of correlation between the values of the particular parameters was established by determining the values of linear correlation coefficients r.

Then empirical directional variograms showing the variation in the water quality parameters along the well depth axis, taking into account the terrain elevation and longer time periods (whereby the character and degree of the variation could be comprehensively described), were determined (2012) [3]. Thanks to the approximation of the directional variograms by means of theoretical analytical functions (called geostatistical models) ordinary (block) kriging could be applied to estimate averages Z* of the studied parameters, with minimum estimation variance σk2 (2012) [3], (2013) [5]. In order to present a general background for the variation in the quality parameters the quick (inverse distance squared) interpolation technique was used (2012) [3]. The unique kriging neighbourhood, i.e. the sample searching subarea, was taken into account. The investigated water intake area near Kłodzko was covered with a grid of rectangular elementary blocks. Different elementary grid dimensions were used in the estimation and interpolation of the topographic parameter, the hydrogeological parameters and the underground water quality parameters. The total number of elementary fields for the centres for which averages Z* and interpolated Z* values were estimated ranged from 51840 to 464640, depending on the spatial analysis variant.

The multidirectional statistical analyses of the variation in the quality parameters indicated certain regularities in the variation of underground water quality and made it possible to determine the trends in this variation (2012) [3], (2013) [4], (2013) [5]. When analyzing the behaviour of the averages of the parameters in the years 1977-1999 one can notice random fluctuations in Fe content and discern a tendency towards periodic changes in Mn content and oxidation capacity (2012) [3], (2013) [4]. It has been found that the average Fe and Mn content in the underground water in the water intake area near Kłodzko in the years 1977-1999 significantly exceeded the standard values for these parameters, i.e. Fe content  < 0.20 [g/m3] and Mn content  <  0.05 [g/m3]. The estimates of the basic statistics for the underground water quality parameters  determined on the basis of the data for the 22 wells  located in the water intake area for the period 1977-1999 indicate extremely high variation in Fe content, very high variation in Mn content and high variation in oxidation capacity. An analysis of the values of variation coefficients V for the particular parameters shows very high variation in Fe content and in nitrate anion NO3- content. Also high variation in Mn content and ammonium ion NH4+ content, for which identical values of coefficients V were obtained, is notable. Temperature C and pH redox potential change very little. The estimates of maximum, average and minimum Fe and Mn content values for the 14 wells (2011) indicate that the standard values for these parameters (Fe content < 0.20 [g/m3], Mn content < 0.05 [g/m3]) are significantly exceeded (2013) [3]. The maximum ammonium anion NH4+ content in the water is close to the standard value, i.e. NH4+ content < 0.50 [g/m3]. In the case of the other analyzed parameters: nitrate anion NO3- content [50gNO3/m3], phosphate anion PO4-3 content [gPO4-3/m3] (an unnormalized value in drinking water), total organic carbon content [5gC/m3], pH potential and temperature C, the statistics do not show any exceedance of the standard values.

The permissible iron content (Fe > 0.20[gFe/m3],) and manganese content (Mn > 0.05 [gMn/m3]) were found to be considerably exceeded in underground water coming from the 3 new piezometers (2012) [3]. The minimum Xmin - 3.99 [gFe/m3], maximum Xmax – 6.12 [gFe/m3] and average - 5.13 [gFe/m3] Fe content values and the minimum Xmin - 0.21 [gMn/m3], maximum Xmax - 0.46 [gMn/m3] and average - 0.34 [gMn/m3] Mn content values were very high. The variation coefficients V calculated on the basis of the data for only the 3 piezometers indicate low variation in Fe content (V – 17.08%) and average variation in Mn content (V – 29.92 %). In the case of other parameters (Ca, Mg, Pb, sulphate, Zn, hydrogencarbonate, chloride and fluoride content and electrical conductivity) the obtained coefficients V indicate low variation of the above parameters (V: 3.15-10.14%). The coefficients V of such parameters as K content, permanganate index (oxidation capacity) show low, but close to average, variation (V: 16.26-17.96%). The other analyzed parameters, such as (total, carbonate and non-carbonate) hardness and solute content have low coefficients V (V: 5.13-7.64 %) or minimal coefficients V – as in the case of total Cr and pH reaction (V: 1.48-1.91 %). The coefficients V for Cd and Cu content indicate average variation for these elements (V: 20.87-25.53%).

The estimates of the basic statistics for the quality parameters of the underground water in the intake area near Kłodzko, based on the data for jointly the 14 wells and the 3 piezometers, indicate the extremely high variation in Fe content (V: 169.19%), high or average variation in Mn content (V: 67.00%) and very low variation in water pH (V: 5.89%) (2012) [3], (2013) [4]. The maximum Fe content of 6.12 [gFe/m3] and the average Fe content of 1.13 [gFe/m3]are considerably exceeded and in the case of Mn content, not only the above statistics - Xmax =1.28 [gMn/m3] and average = 0.36 [gMn/m3], but even the minimum value of 0.19 [gMn/m3] is exceeded. The estimates of the statistical parameters, based on the data for jointly the 14 wells and the 3 piezometers, were undoubtedly influenced by the results of the chemical analyses of Fe and Mn content in the water, performed for the 3 piezometers. The Mg content, the heavy metal (Cd, Cr and Pb) content, the chloride content, the fluoride content and the sulphate anion SO4-2 content in the water coming from the 3 piezometers are much lower than the respective permissible values. This also applies to such parameters as electrical conductivity, permanganate index and total hardness, determined for this water.

The results of chemical determinations of the surface waters in the Bystrzyca Dusznicka river and the Kłodzko Neisse river (23.01.2012) indicate that the total Fe content (the Bystrzyca: Fe – 2.76 [gFe/m3], the Neisse: 4.18 [gFe/m3] exceeds very much the permissible value (Fe < 0.20 [g/m3]) in both rivers (2012) [3]. One should note the extremely high Fe exceedances in the Neisse water. The Mn content (the Bystrzyca: Mn – 0.06 [g/m3], the Neisse – 0.05 [g/m3] in the Neisse water is the same as the permissible value (Mn  < 0.05 [g/m3]). It should be noted that the Mn content in the Bystrzyca water only slightly exceeds the permissible value. The values of the heavy metal content in the surface waters of the two rivers (the Kłodzko Neisse and the Bystrzyca Dusznicka), i.e. Cu content (0.0011 – 0.0014 [g/m3], Pb content (0.0096 – 0.0098 [g/m3), Cd content (0.0011 – 0.0013 [g/m3]  and total Cr content (0.0094 – 0.0095 [g/m3]), are substantially lower than the permissible values. The other quality parameters determined for the surface waters of the Bystrzyca and the Neisse, such as electrical conductivity (25C - 168-306 [µS/cm], chloride content - 5.01-20.38 [g/m3], fluoride content (0.222-0.304 [g/m3]), sulphate anion SO4-2 content (17.08-24.90 [g/m3]), permanganate index (4.14-4.24 [gO2/m3]) and total hardness (68.9-139.4 [gCaCO3/m3], do not exceed the permissible values (2012) [3].

The investigations of the correlation between the underground water quality parameters, based on data for 2011, indicate that the highest positive correlation is between (in the following order): Fe content and ammonium ion NH4+ content (r = 0.89), Fe content and Mn content (r = 0.68), Fe content and temperature (r = 0.71), Mn content and total organic carbon content (r = 0.69), ammonium ion NH4+ content and temperature (r = 0.85), and ammonium ion NH4+ content and phosphate anion PO4-3 content ( r = 0.76) (2012) [3], (2013) [4]. A slightly weaker positive correlation exists between: Fe content and phosphate anion PO4-3 content (r = 0.60), phosphate PO4-3 anion content and temperature (r = 0.61), Fe content total and carbon content C (r = 0.58), temperature and pH (r = 0.64), as well as Fe content and pH,  ammonium ion NH4+ content and pH (r = 0.55) (2012) [3].

The highest positive coefficients (r) of linear correlation between the different underground water quality parameters for the 3 piezometers were obtained for the correlations between: K content and Fe content (0.9979), K content and solute content (0.8264), Mg content and Pb content (0.9999), Mn content and Zn content (0.8806), Fe content and pH (0.9470), Fe content and solute content (0.7884), and K content and pH (0.9657) (2012) [3]. The highest negative correlation coefficients characterize the correlations between: Mg content and Zn content (-0.9975), Mn content and Mg content (-0.8447), Mn content and Pb content (-0.8515), Pb content and Zn content (-0.9983), Mn content and solute content (-0.9294), Zn content and solute content(-0.6435), and Mn content and pH (-0.7559) (2012) [3]. There is a clear, both positive and negative, correlation between K content and Mn content (-0.5601), Mg content and solute content (0.5874), Mn content and Fe content (-0.5055), Pb content and solute content (0.5977), and Zn content and solute content (-0.6435). High positive correlation coefficients r characterize correlations between: Pb content and Cu content (0.9389), total Cr content and Cu content (0.9800), and Pb content and Cr content (0.8515). There is an evident negative correlation between: Pb content and Zn content (-0.9983), Cu content and Zn content (-0.9573), and Cr content and Zn content (-0.8806). There is a clear positive correlation between Fe content and Cr content (0.5055), and a weaker positive correlation between Fe content and Cu content (0.3236).

The directional variation in the semivariogram function γ(h) values of the parameters becomes visible only over a longer study period. The directional semivariograms of Fe and Mn content, calculated along the time axis (the years: 1977 - 2011) for the underground water intake area near Kłodzko show clear upward trends in the value of variogram function γ(h) (Figures 1 - 2) (2012) [3]. In the case of the Fe content semivariogram, the increase in the value of γ(h) is slower, but clear (Figure 1) and in the case of the oxidation capacity semivariogram is slowest (Figure 5). Stronger variation is visible in the Mn content semivariogram and the latter is much steeper (Figure 2).

The highest estimated averages Z* of Fe content in the underground water characterize the years: 1977 – 0.30614÷0.30629 [gFe/m3], 2005 – 0.308925÷0.308940 [gFe/m3], 2008 – 0.34220÷0.34226 [gFe/m3], while the lowest averages Z* are connected with the years: 1988 – 0.084819÷0.084834 [gFe/m3] and 1991– 0.075627÷0.075633 [gFe/m3] (Figure 3) (2012) [3]. The maximum estimated averages Z* of Mn content in the underground water characterize the years: 1977 – 0.52935÷0.5259 [gMn/m3], 1988 – 0.60760÷0.60766 [gMn/m3], 1991 –  0.60785÷0.60791 [gMn/m3] and 2002 – 0.50202÷0.50205 [gMn/m3], while the lowest averages Z* characterize the years:  1994 – 0.362709÷0.362724 [gMn/m3] and 1997 – 0.365084÷0.365099 [gMn/m3] (Figure 4) (2012) [3].

The highest estimated averages Z* of oxidation capacity in the underground water are related to the years: 1984 – 1.873899÷1.873905 [gO2/m3], 1988 – 1.896985÷1.897000 [gO2/m3] and 1998 – 1.807260÷1.807263 [gO2/m3], while the lowest averages Z* characterize the year: 1981 – 1.501546÷1.501561 [gO2/m3] (Figure 6) (2012) [3].

Figure 1: Directional semivariogram (D-90) of  Fe content [gFe/m3]2  in underground water in Klodzko intake area; years 1977–2011.

Figure 2: Directional semivariogram (D-90) of Mn content [gMn/m3]2 in underground water in Klodzko intake area; years 1977–2011.

Figure 3:  Block diagram of estimated averages Z* distribution of Fe content  [gFe/m3] in underground water in Kłodzko intake area; years 1977–2011.

Figure 4:  Block diagram of estimated averages Z* distribution of Mn content  [gMn/m3]  in underground water in Kłodzko intake area; years  1977–2011.

Figure 5: Directional semivariogram (D-90) of oxidation capacity [gO2/m3] in underground water in Klodzko intake area; years 1977-2011.

Figure 6: Block diagram of estimated averages Z* distribution of oxidation capacity [gO2/m3] in underground water in Klodzko intake area; years 1977 – 2011.

Some traces of periodic variation can be discerned in the directional Fe content and Mn content semivariograms (Figures 7 – 8) calculated along the wells depth axis for the underground water intake area near Kłodzko (2012) [3]. Generally, these are short-term variation in the value of semivariogram function γ(h). In the light of the estimates made using ordinary kriging and the inverse distance squared technique it becomes apparent that maximum estimated averages Z* of Fe and Mn content in the underground water generally occur in the SW part of the considered water intake area in the elevation range of 290.55-291.45 m a.s.l. (2012) [3]. Two elevated Fe content levels: a weaker one (3.50-4.50) and a stronger one (4.50-5.00) [gFe/m3], with an envelope of Fe content reaching 3.50-4.50 [gFe/m3], have been identified (Figure 9). In the case of Mn content, only one level of elevated Mn content, ranging from 0.75 to 0.80 [gMn/m3], with an envelope of Mn content reaching 0.65-0.75 [gMn/m3] has been identified (Figure 10).

Figure 7: Directional semivariogram (D-90) of  Fe content [gFe/m3]2  in underground water in Klodzko intake area; 14 wells + 3 piezometers.

Figure 8: Directional semivariogram (D-90) of  Mn content [gFe/m3]2  in underground water in Klodzko intake area; 14 wells + 3 piezometers.

Figure 9: Block diagram of estimated averages Z* distribution of Fe content [gFe/m3] in underground water in Klodzko intake area; 14 wells + 3 piezometers.

Figure 10: Block diagram of estimated averages Z* distribution of Mn content [gMn/m3] in underground water in Klodzko intake area; 14 wells + 3 piezometers.

If data coming only from 14 wells have been taken into calculations one elevated Fe content level: a stronger one (0.80-0.90) [gFe/m3] and a weaker making an envelope of Fe content reaching 0.60-0.70 [gFe/m3], have been identified (Figure 11). In the case of Mn content, only one level of elevated Mn content, ranging from 0.64 to 0.74 [gMn/m3], has been identified (Figure 12).

Figure 11: Block diagram of estimated averages Z* distribution of Fe content [gFe/m3] in underground water in Klodzko intake area; 14 wells.

Figure 12: Block diagram of estimated averages Z* distribution of Mn content [gMn/m3] in underground water in Klodzko intake area; 14 wells.

Owing to the use of the hydrogeochemical space-time model of the variation in the quality parameters of the underground water in the Quaternary formations precise characteristics of the parameters have been obtained for the entire water intake area for the years 1977-2012 (2012) [3], 2013 [5]. The 3D geostatistical model enables the interpolation, modelling and estimation of averages Z*, i.e. the analysis of the quality condition of underground water in a given researcher-defined area over a short and long time span. It becomes possible to determine interpolated values Z* and estimated averages Z* (in a particular space-time point) in the nodes of the 3D grid covering the drainage area of the Kłodzko Region.

By comparing the results yielded by the geostatistical techniques their effectiveness in determining estimated averages Z* and interpolated averages Z* of the underground water quality parameters  was assessed and the advantages of the ordinary (block) kriging procedure used for this purpose became clearly apparent (2012) [3]. Databases containing the grid data yielded by the multidirectional analyses, e.g. the values of coordinates X, Y and Z, averages Z*, estimation standard deviation k and other parameters calculated in the nodes of the spatial elementary network, were created. The content of the databases can be used in further analyses and to obtain grid cross sections.

References

[1] Biedroń I., Świderska-Bróż M., Traczewska T., Trusz.-Zdybek A., Namysłowska-Wilczyńska and Wolska M. (2013), Preliminary assessment of the quality of underground waters in a selected area of the Kłodzko Valley (in Polish). Journal: Instal, Teoria i Praktyka w Instalacjach, monthly,  6 (341), pp. 41-44, Warszawa.

[2] Mądrala M. (2001), Mądrala M. (2001), Hydrogeochemical valorization of environments of river valleys for the needs of underground waters exploitation. Współczesne Problemy Hydrogeologii, pp. 357–364. Wrocław.

[3] Namysłowska-Wilczyńska Barbara (2012), Hydrogeochemical model for underground water intake area in Kłodzko (in Polish). SPR series report no. 1/2012, Institute of Geotechnics and Hydrotechnics at Wrocław University of Technology, March 2012. Text and Attachment. Development Project no. 09-0036 – 10/2011, funded by the National Centre for Research and Development, entitled “The Technology of the biochemical remediation and storage of surface waters in underground hydrogeological structures for municipal water intakes in river valleys”; Research task 3 entitled “Identification and hydrogeochemical models of the Kłodzko Drainage area”, p. 279, Wrocław.

[4] Namysłowska-Wilczyńska Barbara (2013), Geostatistical hydrogeochemical 3D model for  Kłodzko underground water intake area. Part I. Estimation of basic statistics on quality parameters of underground waters. Studia Geotechnica et Mechanica Vol. XXXV, No. 1, pp. 157-181,Wrocław.

[5]  Namysłowska-Wilczyńska Barbara (2013), Estimation of underground water quality parameters for the water intake area near Kłodzko by means of ordinary kriging (in Polish). Monograph, XX KK KOWBAN’ 2013 National Conference “Computer Support of Research”, 23-25.10.2013, pp. 215–225. Polanica Zdrój, Poland.

These are selected results of research carried out as part  the National Centre for Research & Development grant No. N09-0036 – 10/2011, entitled “The technology of the biochemical remediation and storage of surface waters in underground hydrogeological structures for municipal water intakes in river valleys” funded by the Ministry of Science and Higher Education in Warsaw. Research task 3 entitled “Identification and hydrogeochemical models of the Kłodzko Drainage Area (2011-2012)” (2012) [3].

The statistical and geostatistical analyses were carried out using selected computing programs included in the ISATIS software package – version 2013.3, dongle for Isatis (Geovariances & Ecole Des Mines de Paris in Avon Cedex, France) (Isatis 2001).

The dynamic water table is the underground water level (with the terrain surface elevation taken into account) after the well is turned on, i.e. during the operation of the pump. The water abstraction depth is the difference between the terrain elevation and the dynamic water table level (2012) [3].