Automated Screening of Spatio-Temporal Anomalies in Long Term / Large Scale Air Quality Observation Time Series

Oliver Kracht1 & Michel Gerboles2

1 European Commission - Joint Research Centre

Institute for Environment and Sustainability

Air and Climate Unit

Via E. Fermi 2749, I-21027 Ispra, Italy

E-mail: oliver.kracht@jrc.ec.europa.eu

2 European Commission - Joint Research Centre

Institute for Environment and Sustainability

Air and Climate Unit

Via E. Fermi 2749, I-21027 Ispra, Italy

E-mail: michel.gerboles@jrc.ec.europa.eu

Abstract. We are presenting the closer evaluation of a consolidated screening tool for the automatized recognition of anomalies in air quality monitoring data, which considers both attribute values and spatio-temporal relationships. Application examples for the identification of anomalies within the AirBase 2001-to-2010 time series of PM10 background station datasets are demonstrated. Furthermore, sensitivity analyses and validation approaches using synthetic datasets are being discussed. The implemented method is of significant interest as the basis of a data quality screening system.

Keywords. Air pollution, air quality, monitoring, spatial statistics, spatio-temporal outlier, quality control, harmonization

1 	Introduction

Spatial or temporal outliers in air quality datasets are stations or individual measurements which differ significantly from other recordings within a given spatio-temporal neighbourhood. Such abnormal values can be defined as being extreme compared to their neighbours, even though they do not necessarily require to differ significantly from the statistical distribution of the entire population. The identification of such outliers can be of interest as the basis of data quality control systems when several contributors report their measurements to the collection of larger datasets. Beyond this, it can also provide a simple solution to investigate the consistency of station classifications. From another point of view, the primary research interest may also be directed towards the anomalies themselves. In such cases, for example, an outlier detection method can be used as a tool to identify irregular emission events (e.g. air pollution by the influence of fires, wind erosion events, or other accidental situations).

2 	Methodology

We are proposing an adapted method for outlier screening based on a modification and further extension of the existing “Smooth Spatial Attribute Method”, which was first developed for the identification of outlier values in networks of traffic sensors. The original methodological foundations, theoretical background, and applications of this method in traffic research have been demonstrated by Lu, Chen & Kou (2003) and by Shekhar, Lu & Zhang (2003).

For the purpose of investigating ambient air quality datasets, our adapted method was based on the definition of a spatio-temporal neighbourhood for each air quality measurement station, corresponding to a domain limited by time (e.g., +/- 2 days) and distance (e.g., +/- 1 spherical degrees) around the individual location of each monitoring site. It is assumed that within such a given spatio-temporal domain, in which the attribute values of neighbours have a relationship due to the emission, transport and reaction of air pollutants, abnormal values can be detected by extreme values of their non-spatial attributes compared to the non-spatial attribute values of their neighbours. This comparison basically requires a spatio-temporal smoothing, i.e. a specific rule by which data points are averaged within a neighbourhood. The calculation of such reference basis has the effect of a low pass filter, meaning that high frequencies of the signal are removed from the data while preserving low frequencies. In this context, the choice of an appropriate smoother function is of particular importance.

In the following description of our algorithms we are using a modification of the terminology that has originally been outlined for the definitions for the S-outlier concept by Shekhar, Lu & Zhang (2003). We are starting from the consideration that a network of air quality monitoring stations constitutes a collection of spatially referenced objects whose attributes are (i) their locations (spatial attributes, denoted as x), and (ii) the corresponding pollutant measurement values (non-spatial attribute values, denoted as f(x)). Furthermore, for each monitoring station a set of n spatio-temporal neighbours N(x) can be identified, and whose spatio-temporal and non- spatio-temporal attributes are denoted as y (locations) and f(y) (pollutant measurement values).

Within each neighbourhood N(x), several measurements f(yi) of the same compounds, performed at different spatio-temporal locations yi, are available. After a log-transformation of non-Gaussian data, we compute (i) the weighted mean of these measurement values within the neighbourhood of x according to equation (1), and then (ii) the difference S(x) between the pollutant concentration f(x) and the weighted mean of the corresponding neighbourhood values according to equation (2). Suitable methods suggested for the calculation of the weighting factors wi are the inverse squared normalised Euclidean distance or the inverse squared Mahalanobis distance.

(eq 1)

(eq 2)

Within each neighbourhood, the S(x) values of the central station are standardised to centre data at 0 with a standard deviation of 1 using equation (3). In this equation  is the weighted average and  is the weighted standard deviation of the S(xi) values of all n stations within the neighbourhood of x. Note that each of these S(xi) values must be calculated from its own neighbourhood set N(xi), which is usually different from N(x).

(eq 3)

From this step, a time series of ZS(x) values is obtained for each monitoring station. Finally, the test statistics for detecting a spatio-temporal outlier, given in equation (4), searches for ZS(x) values that are exceeding or undercutting the upper or lower limit value of θ, which is defined as a reference basis θref  plus / minus a predefined threshold θconf.

(eq 4)

In this formulation the choice of θconf controls the confidence level of the outlier screening. Example given, assuming the theoretical case of f(x) being the representation of a pure white noise Gaussian random field, a confidence level of 95% will lead to a predefined threshold of θconf  = 1.96. In other words, the expected number of S-outliers identified from a pure white noise Gaussian random field would then be 5%. However, the effective confidence level resulting from real world data will actually depend on the spatiotemporal correlation of the data field.

We have chosen to calculate of the reference basis θref by applying a Kolmogorov-Zurbenko (KZ) filter to the individual ZS(x) time series of each station (equation (5)). In the current applications, a KZ filter with a window size of m = 5 days and k = 3 iterations proved most suitable.

(eq 5)

In this way, we are allowing for long-term trends and natural spatial variations in the air pollutions properties not to be unwontedly detected as an indicator of anomaly. Alternatively, in order to more strictly consider the full spectrum of possible spatial and temporal anomalies, one would prefer θref to be set to zero in this screening step. The differences of these two approaches will be compared.

For safety of the conclusions, different quality controls are applied in the last outlier thresholding step:

- In the case of |ZS(x)| exceeding a value equal to θconf, this ZS(x) value is not taken into account for the calculation of the KZ-filtered series θref = KZ(ZS(x)).

- A minimum number of 3 datapoints is required within a moving window to calculate a valid KZ-filter value KZ(ZS(x)).

- Finally, outliers are only flagged when the reference neighbourhood contains a minimum number of 20 data points.

For each individual point in the time series, calculations are discontinued if any of these conditions is not satisfied. In consequence, the basis for outlier thresholding can be unavailable for some records (such data points are consequently labelled as “non verifiable”).

To better address the shortcomings being caused by control step 3, a dynamic neighbourhood expansion is being used: Starting from an initial trial where the spatio-temporal domain limits of each neighbourhood are set to +/- 1 spherical degrees and +/- 2 days, the algorithm applies a stepwise expansion of in maximum five times of the base extend in time for those cases where insufficient datapoints (e.g., less than 20 individual records) are available. In space dimensions no expansion is allowed.

3 	Application studies

The screening method was tested by using particulate matter PM10 datasets obtained from the European air quality database AirBase. AirBase is maintained by the European Environmental Agency (EEA) that systematically receives measurements of ambient air pollution at more than 6000 monitoring stations from over 30 countries. The quality of these data depends on the chosen method of measurements and quality assurance and quality control (QA/QC) procedures applied by each country. Furthermore, the data in AirBase have undergone additional quality control steps during the upload of the data from the Member States to the EEA database.

Computations presented in this manuscript refer to data queries performed with our in-house postgreSQL database which we build from the official XML files data-repository of AirBase provided at the EEA webpages. Measurements were extracted from AirBase v. 7, while the list of stations and station meta-data came from Airbase v.4. All simulation codes were developed and prototyped in the R environment (R Core Team, 2013).

Taking into account the high spatial variability of PM10 concentrations around industrial and traffic stations, it was decided to apply the screening method solely to stations of background type. In particular we used daily average PM10 records categorized as “background rural”, “background urban“, and “background suburban“ for the decade 01 Jan 2001 to 31 Dec 2011 and from five selected countries (Germany, France, Great Britain, Italy and Spain). Preliminary assessment of the data revealed a log normal distribution pattern of the raw measurement values, and it was chosen to apply a log10-transformation in the first step. The optimal choice of this simple log-transformation was confirmed by a profile likelihood investigation for a Box-Cox transformation model being applied to the data.

4 	Results

Figure 1 illustrates an example of the screening tool being applied to a 10 years AirBase PM10 time series (exemplarily demonstrated for Airbase stations DEBB053). The outcome of such evaluation is a time series of calculated ZS(x) values, a time series of accepted threshold limits, and an identification of abnormal data points, which are exceeding these criteria.

Figure 1: Time series of ZS(x) values for Airbase station DEBB053.

Summary statistics of aggregated results obtained from processing the 2001 - 2010 AirBase v.7 records of daily PM10 values for the background stations of DE, ES, FR, GB, and IT are summarized in Figure 2. In average, the number of identified abnormal data points was ranging between 4.4% and 16.9% of the records within an individual country dataset, whereas between 1.6% and 26.3% of the records per country have been classified as non-verifiable. It needs to be taken note of that this group of non-verifiable values does also include the records of such stations which could not have been evaluated at all over the full time period, because of the general lack of neighbourhood station in their proximity (e.g., the Great Britain stations ranked 55 to 68 in Figure 2). A comprehensive catalogue of decadal time series plots of daily PM10 abnormal values for all background stations of DE, ES, FR, GB, and IT can be obtained from Kracht and Gerboles (2013).

Figure 2: Summary statistics obtained from the screening of the 2001 - 2010 AirBase v.7 records of daily PM10 values for the background stations of DE, ES, FR, GB, and IT. Note that the percentage of abnormal records and unverified records is related to the full number of records.

5 	Sensitivity analysis

The choice of values for the different design parameters of the method will naturally influence the outcome of the outlier screening. This includes for example (1) the spatio-temporal limits of the neighbourhood domain, (2) the threshold value θconf used in equations 4 and 5, (3) the upper limit value for including individual ZS(x) values into the KZ-filtered time series, and (4) the moving window width used for the KZ-filter in the computation of θref = KZ(ZS(x)). Tuning these parameters into the direction of “strict” values (e. g., lower thresholds) obviously causes a larger number of abnormal values (and a different amount of unverified records) in the evaluation outcomes.

In the following we are exemplarily demonstrating a sensitivity analyses for the aforementioned cases (3) and (4). The analysis was based on the background stations records of daily PM10 values from 2006 / 2007.

For the point (3), the z-test for detecting abnormal values (equation 4) is based on the computation of a KZ-filtered time series. Individual ZS(x) values are included into this series provided that their values do not exceed a predefined threshold which is currently set to 1.96. All |ZS(x)| exceeding a value of 1.96 are discarded from the computation. This produces unverified records when several consecutive ZS(x) are rejected, hence restricting a continuous calculation of KZ(ZS(x)).

Figure 3 illustrates the influence of this threshold for accepting ZS(x) values. Tuning this parameter in direction of “strict” values (low threshold) causes a large number of unverified records in the evaluation. At the same time, such a “strict” threshold implies an increased number of identified outliers. This is explained by the fact of a then resulting smoother reference basis θref (equations 4 and 5), which is less strongly moving towards outlying values which are occurring locally in time.

Note also that within the range of possible choices of this threshold parameter, the value of 1.96 (our current default), is close to the point where the responding outlier content reveals its highest variation. Thus, the identified outlier content is strongly sensitive to the choice of this parameter.

For the point (4), the effect of the width of the moving time window of the KZ-filter (m) on the number of detected outlier values was studied. In these simulations only odd values of m where tested, in order to ensure that moving windows were always symmetrical around the central point. In addition, we assumed that for any chosen window width the actual percentage of required minimum number of valid ZS(x) for partial calculations of the KZ filter was set to 60 %.

Figure 4 indicates that the time window of the moving average should not be set to values lower than 5 days in order to avoid a strong decrease of the percentage of detected outliers. Conversely, for time window width over 5 days, only a slight increase of the percentage of detected outlier values takes place.

Figure 4 also displays the limiting value against which the percentage of detected outliers is converging for an infinitely large window size m. From a conceptual point of view, such infinitely large window size m completely removes the time invariant spatial outlierness component, but puts full consideration onto the temporal outlierness within an individual station time series (outlierness as compared to the time series overall mean). The time variant spatial outlierness effects would also still be considered. However, this removal of the time invariant spatial component is usually not considered desirable. Due to the instability of weather conditions over longer time spans, the overemphasis of the (long-term) temporal outlierness component is typically neither desirable.

For data quality screening applications the time window in days should therefore be rather short. We have chosen a default value of 5, as this seems to be a good compromise over stable screening and not indicating to many abnormal values due to false positives.

Figure 3: Sensitivity analysis on the effect of the upper limit value for including individual ZS(x) values into the KZ-filtered time series computation of θref = KZ(ZS(x)). Note that in the current version of our code this limit value defaults to 1.96. The analysis was based on the 2006 - 2007 records of daily PM10 values for background stations.

Figure 4 (follows on next page): Sensitivity analysis on the influence of the moving window width m used for the KZ-filter in the computation of  θref = KZ(ZS(x)). Note that in the current version of our code this value defaults to 5 days. The analysis was based on the 2006 / 2007 records of daily PM10 values for background stations.

6 	Validation by using synthetic datasets

Particular focus of our current investigations is directed towards the quantitative description of the filter efficiency and the effective selectivity of the outlier screening method. Example given, assuming the theoretical case of f(x) being the representation of a pure white noise Gaussian random field, a confidence level of 95% will lead to a predefined threshold of θconf = 1.96. However, the effective confidence level resulting from real world data actually depends on the spatiotemporal correlation of the data field.

Table 1 provides some basic statistical properties obtained from the 2006 / 2007 background stations records of daily average PM10 values. Based on these characterizations, a set off 30 surrogate datasets was produced. We assumed a spatial domain extend of 8 by 8 degrees, and a temporal extend of 2 years. 1089 virtual monitoring stations were placed on a regularly spaced grid with grid spacing of 0.25 degrees. In a first step, for each of these monitoring stations individual synthetic time series of daily average PM10 values were produced from log-normally distributed random numbers. For the evaluations presented here, all stations were considered as being independent and no spatial autocorrelation or temporal autocorrelation was introduced into the random fields. In a second step, 10 different linear trend scenarios were superimposed to the data. In specific, west to east trends of 0, 1, 2.5, 5, 10, 15, 20, 50, 100 and 200 µg m-3 deg-1 were used, with three repetitions each. The aim of this numerical experiment was to investigate if such a superimposed trend pattern might impair the accuracy of the outlier detection.

Table 1: Basic statistics of the 2006 / 2007 AirBase v.7 records of daily average PM10 values of background stations from 5 different country. Note that the unit of the raw measurement data is µg/m3. Rows 1 to 3 are the empirical mean, standard deviation and skewness of the raw data, whereas rows 4 to 6 are presenting the same empirical parameters obtained from data on the log scale.

|  |  | DE | ES | FR | GB | IT | Mean |
| --- | --- | --- | --- | --- | --- | --- | --- |
| (1) | Mean (raw data) | 21.34 | 28.57 | 22.59 | 22.2 | 35.26 | 25.99 |
| (2) | Stdv (raw data) | 13.78 | 17.83 | 13.47 | 11.52 | 25.54 | 16.43 |
| (3) | Skew (raw data) | 2.5 | 2.61 | 2.38 | 2.2 | 1.91 | 2.32 |
| (4) | Mean (ln data) | 2.89 | 3.19 | 2.97 | 2.98 | 3.35 | 3.08 |
| (5) | Stdv (ln data) | 0.59 | 0.57 | 0.55 | 0.49 | 0.65 | 0.57 |
| (6) | Skew (ln data) | -0.2 | -0.69 | -0.02 | -0.67 | -0.35 | -0.39 |

Table 2 summarizes the configuration details and compares the results obtained. Considering that the predefined threshold of θconf was set to 1.96, an unbiased estimate of the outlier content in the zero-trend scenario should have delivered a value of 5%. The results of the stochastic simulation are falling close to it, though there is a slight underestimation persisting. In fact, similar simulations with Gaussian data (not shown here) provided mean outcomes centering around 5%. More important it is to notice, that the estimated outlier content remains acceptable stable for the following 2.5, 5, 10, 15, 20 and 50 µg m-3 deg-1 trend scenarios. Only the 100 and 200 µg m-3 deg-1 trend scenarios are strong enough to cause a relevant disturbance of the outlier detection accuracy.

7 	Conclusions

Experiences obtained with this further developed version of our outlier screening tool are promising. The presented algorithms are designed to detect local outliers which are characterized by a sharp spatio- temporal non-stationarity of their attribute values (e.g., air pollutant concentrations). However, attribute values which share a smooth spatio-temporal covariance structure, will not wrongly be categorized as outliers. Example given, it has been demonstrated that global trends, which are likely to occur in real-world data, do not strongly affect the accuracy of the outlier screening. In these examples, the order of magnitude of those global trend scenarios which first had a relevant influence on the accuracy of outlier detection can in fact be excluded to occur in real data, where the method is considered to deliver robust results. This makes it a promising technique for the analysis of background type monitoring stations. In contrary, the inherent spatial non-stationarity of the non-background sites makes it basically difficult to be applied at.

The identification of abnormal values can be of interest as the basis of data quality control systems. Beyond this, it can also contribute an alternative solution to investigate the consistency of station classifications. Furthermore, in other type of applications, the anomalies themselves may be in the primary focus of interest. In such cases, for example, our method is anticipated to a serve as a useful tool to detect irregular air pollution emission events (e.g. the influence of fires, wind erosion events, or other accidental situations).

Table 2: Numerical simulation results of test runs performed with 30 different synthetic datasets. Note that synthetic data were produced from log-normally distributed random numbers, whereas the specifications of the mean, the standard deviation and the superimposed trend are in unit µg m-3 of back-transformed data on the normal scale.

|  | Repetition 1 | Repetition 2 | Repetition 3 | Mean |
| --- | --- | --- | --- | --- |
| Spatial Extend | 8 deg x 8 deg | 8 deg x 8 deg | 8 deg x 8 deg | - |
| Temporal Extend | 2 years | 2 years | 2 years |  |
| Grid Spacing | 0.25 deg | 0.25 deg | 0.25 deg | - |
| Number of Stations | 1089 | 1089 | 1089 | - |
| Mean norm-scale | 26 µg m-3 | 26 µg m-3 | 26 µg m-3 | - |
| Stdv norm-scale | 16 µg m-3 | 16 µg m-3 | 16 µg m-3 | - |
| Superimposed Trend | Identified Outlier Content |  |  | Mean |
| 0 µg m-3 deg-1 | 4.68% | 4.68% | 4.70% | 4.69% |
| 1 µg m-3 deg-1 | 4.53% | 4.57% | 4.54% | 4.55% |
| 2.5 µg m-3 deg-1 | 4.50% | 4.52% | 4.49% | 4.50% |
| 5 µg m-3 deg-1 | 4.56% | 4.58% | 4.56% | 4.57% |
| 10 µg m-3 deg-1 | 4.69% | 4.66% | 4.69% | 4.68% |
| 15 µg m-3 deg-1 | 4.78% | 4.75% | 4.75% | 4.76% |
| 20 µg m-3 deg-1 | 4.76% | 4.72% | 4.75% | 4.74% |
| 50 µg m-3 deg-1 | 4.54% | 4.55% | 4.54% | 4.54% |
| 100 µg m-3 deg-1 | 4.08% | 4.03% | 4.08% | 4.06% |
| 200 µg m-3 deg-1 | 3.28% | 3.26% | 3.25% | 3.26% |

Due to limited space available for this extended abstract, only one type of analysis with synthetically data could be shown and discussed. We propose that if selected for a paper publication in one of the special issues, we will add more simulations with extended discussion, including single spot anomalies, nonlinear trends and discontinuities.

References

Lu, CH.-T., Chen, D. and Kou, Y, (2003), Detecting Spatial Outliers with Multiple Attributes, Proceedings of the15th IEEE International Conference on Tools with Artificial Intelligence (ICTAI'03), IEEE 2003.

Shekhar, S., Lu, CH.-T., and Zhang, P., (2003), A Unified Approach to Detecting Spatial Outliers, GeoInformatica, Vol. 7 No. 2, 139-166.

Kracht, O. and Gerboles, M., (2013), Screening of Spatio-Temporal Anomalies in Long-Term / Large Scale Air Quality Monitoring Time Series, JRC Technical Reports 87445, EUR 26462 EN, ISSN 1831-9424 (online), ISBN 978-92-79-35154-9 (PDF), DOI 10.2788/60933.

R Core Team (2013). R: A language and environment for statistical computing. R Foundation for Statistical Computing, Vienna, Austria. URL http://www.R-project.org/.