A geostatistical methodology for the performance evaluation of groundwater quality monitoring networks

Hugo Júnez-Ferreira1, Julián González2, Emmanuel Reyes3 & Graciela Herrera4

1 Maestría en Ingeniería Aplicada, Universidad Autónoma de Zacatecas. Av. Ramón López Velarde 801, Col. Centro,  C.P. 98010, Zacatecas, México. hejunez@hotmail.com

2 Maestría en Ingeniería Aplicada, Universidad Autónoma de Zacatecas. Av. Ramón López Velarde 801, Col. Centro,  C.P. 98010, Zacatecas, México. aguabuena_62@yahoo.com.mx

3 Maestría en Ingeniería Aplicada, Universidad Autónoma de Zacatecas. Av. Ramón López Velarde 801, Col. Centro,  C.P. 98010, Zacatecas, México. jesus_reyes_ic@hotmail.com

4 Instituto de Geofísica, Universidad Nacional Autónoma de Zacatecas. Ciudad Universitaria, Del. Coyoacán 801, C.P. 04510, México City, México. ghz@geofisica.unam.mx

Abstract. A geostatistical based methodology is proposed to evaluate existing groundwater quality monitoring-networks by considering simultaneously the spatial correlation for various physicochemical parameters and the aquifer vulnerability index, using as optimization criterion to minimize a weighted normalized estimate error variance of all the variables. For the vulnerability index determination, the DRASTIC method was chosen as it is one of the most popular to assess the potential of contamination in aquifers. A covariance matrix for each variable is obtained from geostatistical analysis of data, each is normalized dividing all its elements by the parameter variance (the sill of its corresponding variogram model); this is done to initially give the same weight to each parameter, later, depending on the monitoring objectives, weights can be specified during optimization. The static Kalman filter was chosen as the estimation method, it requires an initial estimate and its error covariance matrix. Through a heuristic optimization method, the contribution of each monitoring well in reducing the estimate error variance for all the parameters at pre-defined estimation points is evaluated.

Keywords. Monitoring-network, groundwater quality, geostatistics, Kalman filter, vulnerability

1 	Introduction

Several examples of the use of geostatistics in the design of optimal groundwater quality monitoring-networks can be found in the specialized literature. These works often consider one, and sometimes various physicochemical parameters present in water. Chadalavada et al. (2011) and Li et al. (2011) use spatial correlations between concentrations of a single solute. Lin and Rouhani (2001) designed a different groundwater quality monitoring network for each of the two analyzed contaminants: Trichloroethylene and Tetrachloroethylene. Yeh et al. (2006) employed a multivariate analysis and factorial kriging to design a spatial monitoring network for nine water quality parameters.

Furthermore, besides current solute concentrations, hydrogeological aspects and contamination risks   of aquifers have also been incorporated in the design  by means of the vulnerability index. Dutta et al. (1998) employed vulnerability indexes (calculated with the DRASTIC method proposed by Aller et al. 1987), industry and stream network coverage, and data of some sampled water quality parameters to suggest different monitoring alternatives but in the optimization process only the parameter with larger variance was considered. Narany et al. (2014) presented a methodology that combines, through a Geographic Information System (GIS), a risk map (calculated with vulnerability indexes and land use) and an exceeding groundwater nitrate threshold probability map (obtained with indicator kriging). The highest combined index represents areas with a very strong probability of finding nitrate concentrations and higher relative risk to contamination. The monitoring network was proposed by a visual procedure. Authors concluded that a monitoring netwok design should consider jointly risks and the current state of contamination.

As far as the authors of this paper are aware, there is no method based on geostatistics that involves several water quality parameters and vulnerability index simultaneously on the objective function of an automated process to define wells of an optimal monitoring network.

Herrera (1998) presented a methodology that uses a heuristic optimization method in combination with a Kalman filter to assign the priority order of sampling wells for a single groundwater quality parameter. This methodology requires, among other inputs, a prior covariance matrix of the water quality parameter considered for the design of the monitoring-network, the elements of this matrix are derived from a numetical tranport model. As optimization criterion, a priority order is assigned to each well according to its contribution in minimizing the total variance (the sum of the diagonal elements of the covariance matrix). In more recent works, Herrera et al. (2004), Júnez (2005) and Hergt (2009) modified the methodology for the design of an optimal monitoring network (OMN) for various water quality parameters. Unlike Herrera (1998), they used geostatistical analysis to calcultate the covariance matrices of various water quality parameters. In the optimization procedure, all the water quality parameters carry the same weight, and a joint normalized variance is minimized to assign a priority order to wells.

In this paper, the latter geostatistical methodology is modified to evaluate existing groundwater quality monitoring networks by considering simultaneously in a weighted manner various physicochemical parameters; furthermore, hydrogeologic aspects are incorporated by means of the aquifer vulnerability index correlation. The selected optimization criterion is to minimize the weighted normalized estimate error variance (this will be defined later in this paper).

2 	Materials and methods

The proposed methodology was applied to evaluate a pilot monitoring-network in Calera aquifer, México. Calera aquifer is located at the central part of the mexican state of Zacatecas, where the principal state population centers are located, and a quarter of the state population resides (Figure 1). This aquifer has a horizontal extent of approximately 2,000 km2 and is defined as unconfined in a granular medium that fills a regional graben with thickness varying from 38 m at north to 570 m at the central and southern zones. Preferential flow direction is from south to north. Groundwater represents practically the only source of water to satisfy potable, industrial and agricultural needs in the region.

Due to its importance, contamination of groundwater in the Calera aquifer would lead to a catastrophic scenario. Therefore, it is of paramount importance that the groundwater quality monitoring network be able to anticipate adverse changes in water and at the same time permits the early detection of anthropogenic contaminants entering into the aquifer. The methodology proposed in this paper seeks to incorporate both aspects in the design and evaluation of monitoring networks by considering simultaneously the actual concentration of indicator parameters in water and those zones where the aquifer has a higher susceptibility to be contaminated.

Figure 1. Calera aquifer geological framework, location of the pilot monitoring network, and groundwater flow directions

The objectives of a groundwater monitoring-network depend on the needs of the water resources management organizations. A performance evaluation of a groundwater quality monitoring-network may be carried out by estimating the way in which each monitoring well contributes to minimize the estimate error variance of the sampled indicator parameters at predefined estimation points. If the vulnerability index is included in the evaluation, it is desirable that estimates uncertainty should be minimized at zones corresponding to its highest relative values.

In this paper, the evaluation of each monitoring position is done using a Kalman filter and an optimization method. When working with a monitoring network for various variables, the Kalman filter as used here, requires a prior spatial covariance matrix (calculated through geostatistics) for each of them; the heuristic optimization procedure seeks to reduce the estimate error variance over an area of interest (it could consider contaminated areas, highly vulnerable zones, pollution sources, potable water exploitation zones, recharge zones, or over the entire area that defines an aquifer). To achieve this, it is necessary a spatial set of locations for which estimates are needed (the estimation grid). An alternative to assign more weight in the optimization process to areas with higher priority is to use a denser set of estimation nodes over them.

The Kalman filter

The Kalman filter provides a minimum-variance unbiased linear estimate for the state of a system given noisy data (Jazwinski, 1970). In this paper, we apply the static Kalman filter (KF) formulas presented in Júnez-Ferreira and Herrera (2013) for each analyzed water quality indicator parameter (WQIP) and the vulnerability index in a spatial sense.

One important advantage of the usage of this estimation method (in the way it is employed) is that for evaluating each monitoring position, it is required to solve the system of equations presented in Júnez-Ferreira and Herrera (2013) only once since the state vector and the covariance matrix are updated in all the estimation positions. This could also be done with kriging but could involve heavier computation (Webster and Oliver, 2007).

Monitoring network optimization

The optimization procedure employed in this paper to consider various variables simultaneously with a specific weight associated to each of them is explained below.

Consider the set of all possible monitoring positions M=xiM,i=1,...,Nmp where Nmp is the number of possible monitoring positions and x=x,y∈D, where D is the Euclidean plane. From this set, we want to choose those points that minimize the estimate error variance on the points for which estimates are needed, E=xjE,j=1,...,Nep, where Nep is the number of estimation points.

If in the optimization procedure we employ the covariance matrix as obtained directly from the geostatistical analysis, more weight during the optimization procedure would be given to variables with large variance than to variables with small variance, therefore low-variance variables would be poorly monitored with the resulting monitoring network.

In order to control the weight of each variable during the optimization procedure, we normalized the covariance matrix obtained from the geostatistical analysis for each variable to obtain a correlation matrix (CM). The normalization of each covariance matrix is carried out dividing all its elements by the variance of the variable (the sill of its corresponding variogram model), in this way the principal diagonal for each prior CM will be entirely composed by ones. This is,

CMk0=Ck0C(0)k						          (1)

where CMk0 is the correlation matrix for variable k, Ck0 is the covariance matrix obtained from the geostatistical analysis for variable k, and C(0)k is the sill of the variogram model for variable k.

In this paper, the optimization objective function is the normalized weighted variance of the estimate error, calculated using the following formula:

σNW2=k=1Nvj=1NepWkσj,k2 				                                                   (2)

where σNW2 is the normalized weighted variance, σj,k2 is the normalized estimate error variance of the variable k at the j-th estimation location, obtained from the diagonal of the KF normalized covariance matrix of each variable, Wk is the weight for variable k, Nv is the number of variables considered for the design or the performance evaluation. In the optimization objective function, it is evaluated the contribution of each monitoring point in reducing the estimate error variance for all the variables at all the estimation points.

The problem is optimized sequentially using a successive-inclusions method (Samper and Carrera, 1990), details can be reviewed in Júnez-Ferreira and Herrera (2013).

A good approach to determine the high priority monitoring wells (HPW) is to consider the spatial locations where a 90 % of the maximum possible reduction (MPR) is achieved (Júnez-Ferreira & Herrera, 2013):

MPR=MaxσJT2Nep−MinσJT2Nep                                                                                                   (4)

Vulnerability index

For the vulnerability index determination, the DRASTIC method (Aller et al. 1987) was chosen because it is one of the most popular to assess the potential of contamination in aquifers, this method is based on the assignment of indexes according to local features in a well.

The parameters considered in the DRASTIC acronym are: D (depth to groundwater), R (net recharge), A (aquifer media), S (soil type), T (topography slope), I (impact of the vadose zone) and C (hydraulic conductivity of the aquifer), are evaluated ranging from one (low vulnerability) to ten (maximum vulnerability).

To determine the vulnerability index, each vulnerability parameter is multiplied by its corresponding weight, and results are summed to obtain a final value of the vulnerability index for each analyzed position:

DRASTICINDEX=DrDw+RrRw+ArAw+SrSw+TrTw+IrIw+CrCw		          (5)

where subscripts r and w indicate rating and weight assigned to each parameter, respectively.

3 	Results

Data of arsenic, calcium, sodium, sulfate, and chloride where obtained for 43 wells in a sampling campaign conducted in 2007 for the Calera region. From these positions, 36 correspond to Calera aquifer and they constitute a pilot monitoring-network (PMN), these wells cover approximately in a uniform way the horizontal extent of the aquifer (Figure 1). The vulnerability index was calculated by applying the DRASTIC method in the same positions of the PMN. Statistics of data collected (for the WQIP) and calculated (for the vulnerability index) are shown in Table 1.

Table 1: Summary statistics for considered variables

|  | Arsenic (mg L-1) | Calcium (mg L-1) | Chloride (mg L-1) | Sodium (mg L-1) | Sulfate (mg L-1) | Vulnerability Index (adim) |
| --- | --- | --- | --- | --- | --- | --- |
| Minimum | 0.004 | 56.95 | 2.26 | 13.80 | 6.74 | 84 |
| Maximum | 0.090 | 402.73 | 117.97 | 156.40 | 468.00 | 133 |
| Mean | 0.023 | 108.37 | 13.82 | 47.83 | 51.58 | 119.25 |
| Median | 0.021 | 99.66 | 8.36 | 43.24 | 34.70 | 120 |
| Standard deviation | 0.015 | 60.19 | 19.10 | 25.04 | 83.74 | 7.94 |
| Skewness | 2.134 | 3.54 | 4.22 | 1.96 | 4.14 | -2.75 |
| Kurtosis | 10.069 | 16.58 | 22.37 | 9.34 | 19.42 | 11.10 |
| No. of data | 43 | 43 | 43 | 43 | 43 | 36 |

A geostatistical analysis of the physicochemical parameters sampled in 2007 was previously carried out in Júnez et al. (2013). In this paper, only the analysis was done to vulnerability data; for this case regression analysis was used to remove the detected trend. Parameters of selected variogram models and cross validation results for all the variables considered in the performance evaluation of the pilot monitoring-network are shown in Table 2.

Table 2: Parameters of selected variogram models and cross validation results

| Variable | Model | Model Parameters |  |  | Cross validation results (original variable) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Nugget | Sill | Range (m) | Mean error (mg L-1) | Minimum error (mg L-1) | Maximum error (mg L-1) | SMSE |
| ln arsenic | Spherical | 0.147 | 0.423 | 21 727.80 | -0.0003 | -0.0673 | 0.0257 | 1.278 |
| Calcium | Spherical | 179.84 | 736.01 | 38 127.70 | -1.020 | -51.114 | 53.756 | 1.235 |
| ln chloride | Spherical | 0.20 | 0.60 | 37 000.00 | -0.445 | -49.576 | 13.853 | 1.299 |
| ln sodium | Spherical | 0.03 | 0.25 | 27 233.30 | 0.036 | -52.256 | 48.657 | 1.388 |
| ln sulfate | Spherical | 0.001 | 0.54 | 38 900.00 | 0.585 | -46.392 | 39.130 | 1.000 |
| Vulnerability index (residuals) | Exponential | 4.77 | 14.75 | 47 486.60 | -1.163 | -30.762 | 10.910 | 1.314 |

*SMSE- Squared mean standardized error

The Kalman filter was used to obtain estimates and its associated estimate error variances for all of these variables within the Calera aquifer extent, in Figures 2 and 3 we present only those for arsenic and the vulnerability index, respectively. The highest relative values for the vulnerability index are located at the central and at the northern part of the aquifer, which means that those zones are more vulnerable to be contaminated. An analysis carried out by Reyes (2014) relates these relative high vulnerability values with the presence of nitrates which gives confidence that calculated vulnerability values can help in evaluating how well the monitoring-network can detect anthropogenic contamination.

Figure 2: Maps of a) estimates and b) estimate error variances for arsenic in Calera aquifer using the PMN

Figure 3: Maps of a) estimates and b) estimate error variances for the vulnerability index in Calera aquifer using the PMN

Performance evaluation of the pilot monitoring-network

The objectives for the optimal monitoring-network in Calera aquifer are: 1. Selected wells allow to obtain low-uncertainty estimates of groundwater quality parameters for the entire aquifer, 2. The entry of contaminants into the aquifer should be early detected, and 3. Wells that provide redundant information should be avoided.

The proposed methodology allows consider weights for each variable intervening in the evaluation process; in this way, it is possible to evaluate monitoring preferences established in the objectives of the monitoring-network. In the case study, it was desirable that the monitoring-network had the same probability to notice adverse changes in WQIP (therefore, low uncertainty estimates of actual solute concentration must be obtained) and early detect the entry of contaminants into the aquifer. To achieve this, a 50 % weight was assigned to the WQIP together and the other 50 % to the vulnerability index. Since five WQIP are considered in the performance evaluation, a 10 % weight was assigned to each of them. These weights can change according to monitoring objectives.

Since the largest estimate error variances for the vulnerability index coincide with the highest estimate values for this variable (Figure 3), the way this variable was employed in the proposed optimization procedure permitted to assign a high priority to wells located inside or near to highly susceptible to anthropogenic contamination zones.

The estimation grid (nodes where estimates are required) was defined with squared elements of 1 km side length, this grid encompasses the existing PMN (Figure 4).

Figure 4: Estimation grid and priority order of the HPW

When the optimization method is applied, the order in which the spatial locations are selected in a natural way represents a priority order, because at each round the location that reduces the most the variance (in other words, the location that gives maximum information) is selected. In this way, the selection order is an indicator of how important are the data obtained at those locations in reducing the normalized weighted variance: if the selection order is small, the priority is high. This can be seen clearly in Figure 4, the normalized weighted variance is greatly reduced when choosing the first spatial locations, but as the number of selected locations increases, the amount of the normalized weighted variance reduction decreases.

Figure 5: Normalized weighted variance vs number of spatial monitoring points

It was found that a 90 % of the MPR is achieved when 16 wells of the PMN are used, we called them high priority wells (HPW), the remaining 20 wells are considered to provide redundant information and therefore they are identified as low priority wells (Figure 4). It can be seen, as expected, that wells with the highest priority were selected nearby zones with the highest relative vulnerability.

Finally, estimates and its associated estimate error variances using the HPW were obtained for all the variables. In Figures 6 and 7, estimates for arsenic and the vulnerability index are presented.

Figure 6: Maps of a) estimates and b) estimate error variances for arsenic in Calera aquifer using the HPW

Figure 7: Maps of a) estimates and b) estimate error variances for the vulnerability index in Calera aquifer using the HPW

To compare the estimates obtained with the PMN (36 wells) and the HPW (16 wells), some statistics of differences between estimates and error variances obtained with the KF for both cases are presented in Table 3. In the column of PMN – HPW estimates, the mean absolute differences are shown, it can be seen that differences between estimates for both options are small. The square root mean variance (SRMV) is a measure of the average uncertainty in the estimation grid when employing the PMN or the HPW. The comparison in Table 3 shows the increase in the average uncertainty when employing the PMN and the HPW; it can be seen that when using the HPW, a marginal loose of certainty is obtained compared to the PMN.

Table 3: Comparison of some statistics of the estimates obtained with the Kalman filter for the data from the pilot monitoring network and the high priority wells

|  |  | Arsenic (mg L-1) | Calcium (mg L-1) | Chloride (mg L-1) | Sodium (mg L-1) | Sulfate (mg L-1) | Vulnerability Index (adim) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PMN-HPW (mean absolute differences) |  | 0.0018 | 7.269 | 1.426 | 2.774 | 4.327 | 0.914 |
| SRMV | PMN | 0.0114 | 0.634 | 8.314 | 13.410 | 11.486 | 0.729 |
|  | HPW | 0.0118 | 0.664 | 8.615 | 14.261 | 12.661 | 0.759 |

4 	Conclussions

The proposed methodology considers simultaneously various water quality parameters and the vulnerability index in the performance evaluation of an existing monitoring network, furthermore it could be used for the optimal design or redesign of monitoring networks for tracking the evolution of groundwater quality, considering at the same time vulnerable-to-contamination areas, and the current state of solute concentrations in groundwater. It employs the spatial correlation between data of each variable using spatial covariance models derived from geostatistical analysis.

The performance evaluation showed that 16 of the 36 wells of the pilot monitoring network provide 90 % of the maximum possible reduction of the normalized weighted variance for the selected variables. This implies more than a 50 % of cost reduction with a marginal increase in uncertainty to achieve the monitoring objectives. A maximum increase of 10.2 % of the SRMV values within all the nodes of the estimation grid was obtained for the selected HPW of 16 wells, compared to the SRMV calculated with the PMN of 36 wells.

Areas with large values of total variance after measuring in the monitoring locations indicate that the spatial distribution of that parameter is not represented with certainty in certain zones. This means that new wells should be added to the optimal monitoring network in those zones. To propose the locations of the new wells, potential pollution sources and hydrogeological criteria should be also considered.

The proposed methodology is versatile for including practical hydrogeological criteria since it allows assigning weights to variables involved in the evaluation or in the design of a monitoring network. In this paper, the relative highest vulnerability zones were incorporated in the design by means of its spatial correlation. An alternative step may include weighting spatial positions to consider certain zones of interest.

References

Aller, L., Lehr, J.H., Petty, R. , Bennett, T., & Hackett. G. (1987). DRASTIC: A standardized system to evaluate groundwater pollution using hydrogeologic settings. US Environmental Protection Agency Report 600 2-87 035: 641.

Chadalavada, S., Datta, B., & Naidu, R. (2011). Uncertainty based optimal monitoring network design for a chlorinated hydrocarbon contaminated site. Environmental Monitoring and Assessment, 173, 929-940.

Dutta, D., Das Gupta, A., & Ramnarong, V. (1998). Design and optimization of a ground water monitoring system using GIS and multicriteria decision analysis. Ground Water Monitoring and Remediation, 18(1), 139-147.

Hergt, T. (2009). Diseño optimizado de redes de monitoreo de la calidad del agua de los sistemas de flujo subterráneo en el acuífero 2411 "San Luis Potosí": Hacia un manejo sustentable, Tesis de doctorado, Universidad Autónoma de San Luis Potosí.

Herrera, G. S. (1998). Cost Effective Groundwater Quality Sampling Network Design, Ph. D. Dissertation, University of Vermont.

Herrera, G. S., Júnez-Ferreira, H. E., González, L., & Cardona, A. (2004). Diseño de una red de monitoreo de la calidad del agua para el acuífero Irapuato-Valle, Guanajuato. Memorias del XVIII Congreso Nacional de Hidráulica, AMH, SLP, México.

Jazwinski, A. H. (1970). Stochastic processes and filtering theory. London: Academic Press.

Júnez, H. E. (2005). Diseño de una red de monitoreo de la calidad del agua para el acuífero Irapuato-Valle, Guanajuato. Tesis de Maestría, UNAM, México. Online access http://132.248.9.195/ptd2012/anteriores/0339152/Index.html

Júnez-Ferreira, H. E., & Herrera, G. S. (2013). A geostatistical methodology for the optimal design of space-time hydraulic-head monitoring-networks and its application to the Valle de Querétaro aquifer, Environmental Monitoring and Assessment. 185(4), 3527-3549.

Li, J., Bárdossy, A., Guenni, L., & Liu, M. (2011). A Copula based observation network design approach. Environmental Modelling & Software, 26, 1349-1357.

Lin, Y., & Rouhani, S. (2001). Multiple-Point Variance Analysis for optimal adjustment of a monitoring network. Environmental Monitoring and Assessment, 69, 239-266.

Narany, T., Ramli, M., Aris, A., Sulaiman, W., & Fakharian, K. (2013). Spatial Assessment of Groundwater Quality Monitoring Wells Using Indicator Kriging and Risk Mapping, Amol-Babol Plain, Iran. Water, 6, 68-85.

Reyes, J. E. (2014). Metodología geoestadística que involucra la vulnerabilidad en el diseño de redes de monitoreo de la calidad del agua subterránea: Aplicación Calera, Zacatecas. Tesis de Maestría, Maestría en Ingeniería Aplicada: Orientación en Recursos Hidráulicos, Zacatecas, México.

Samper, F. J., & Carrera, J. (1990). Geoestadística, aplicaciones a la hidrogeología subterránea. Centro Internacional de Métodos Numéricos en Ingeniería, Universidad Politécnica de Cataluña, Barcelona, España.

Webster, R., & Oliver, M. (2007). Geostatistics for environmental scientists. Wiley, UK.

Yeh, M., Lin, Y., & Chang, L. (2006). Designing an optimal multivariate geostatistical groundwater quality monitoring network using factorial kriging and genetic algorithms. Environ Geol, 50, 101-121.