Kriging for data collected over large areas 
of the Earth’s surface

Alexander Gribov1 & Konstantin Krivoruchko2

1 Environmental Systems Research Institute, 380 New York St, Redlands, CA, USA, 92373, agribov@esri.com

2 Environmental Systems Research Institute, 380 New York St, Redlands, CA, USA, 92373, kkrivoruchko@esri.com

Abstract. In this paper, we discuss several kriging methods that employ distances calculated on three-dimensional bodies rather than on the plane.

First, we review four methods for calculating distance between distant locations on the Earth. Next, we discuss kriging over large areas using great arc distances (the shortest distance between two points on the surface of a sphere.) Then we review a more computationally intensive kernel convolution approach on an arbitrarily shaped three-dimensional body. Distances calculated on the hypothetical surface of the earth that coincides with the mean sea level (in practice, on a geoid) are the least distorted, but result in the longest kriging computations. Finally, we discuss empirical Bayesian kriging with chordal distances, which are the distances that light would travel between two points on a transparent globe. This approximation of distance results in the fastest predictions. The empirical Bayesian kriging operates with large data sets by dividing them into subsets and then merging the models from nearby subsets (see Krivoruchko and Gribov, 2014 for a more in-depth explanation.) We show that the empirical Bayesian kriging with chordal distances works well both with global (i.e. the data collected over the entire Earth) and local data (i.e. the data collected in the vicinity of the North Pole.)

This extended abstract illustrates several concepts and models and summarizes the main paper’s results. All mathematical details and additional applications will be provided in the forthcoming full paper.

Keywords. Empirical Bayesian kriging, kernel convolution, geoid, ellipsoid, chordal distance.

1 	Modeling data separated by large distances

Most spatial data analysis models assume the geometry of a flat surface and, therefore, it is strongly recommended to work with projected data instead of using the latitude and longitude coordinates. However, the curved surface of the Earth presents a problem when interpolating and mapping over large areas because map projections distort shapes, areas, directions, and distances between points on the Earth’s surface. Some projections such as Robinson attempt to minimize distortion across the world through some compromise of all those factors. Other projections such as UTM are designed for specific areas of the globe in order to keep the distortion minimal.

The Web Mercator projection used by Google, Bing and ArcGIS Online was designed to project a perfect sphere into a square area and it does a little to minimize distortions. The biggest problem is the heavy vertical and horizontal stretching at extreme latitudes. This is evident from the heavily increased areas of Greenland and Antarctica relative to land masses closer to the Equator as shown in figure 1 at left. The farther away from the Equator, the more distorted distance measurements are.

The most important (empirical) model of the Earth which represents true zero elevation surface, called geoid, is shown in figure 1 at right.

Figure 1: Left: Web Mercator map of the World. Right: Geoid skeleton

Both a sphere and an ellipsoid approximate the shape of the Earth and figure 2 shows that the usage of ellipsoid is an improvement of the distance calculation over the Earth surface in comparison with the usage of a sphere. However, the usage of a sphere is much easier because the distances between points are location independent in contrast to the ellipsoid surface.

Figure 2: The difference between the geoid and sphere (left) and ellipsoid (right), in meters

2 	Kriging on a sphere

To perform kriging on a sphere, it is sufficient to use “great circle” distance metrics to compute distances between points and to use the semivariogram model which is positive definite on a sphere. Among the valid semivariogram models are spherical, stable with shape parameter 1, and K-Bessel (same as Mattern) with shape parameter 0.5, see, for example, Guinness and Fuentes, 2013. Disadvantages of this approach include the following:

- All the allowed standard semivariogram models are not flexible because they change fast at short distances.

- Standard anisotropy option cannot be used.

- There is no theory for IRF-k kriging on a sphere (this is problematic because IRF-k is the default model for empirical Bayesian kriging, the most popular kriging in the ArcGIS Geostatistical Analyst, see Krivoruchko, 2012.)

- There are theoretical problems with the use of local polynomial interpolation (used in the Geostatistical Analyst for data detrending and interpolation with barriers, Gribov and Krivoruchko, 2011.)

- Interpolation using small datasets for very large areas (i.e. the data with very large range of correlation) requires the use of a complex non-stationary semivariogram model with a large number of parameters (Hitczenko and Stein, 2012.)

Some of the above-mentioned issues can be resolved by using the kernel-convolution approach. Construction of the covariance model using kernel-convolution on the unit sphere located at the origin of coordinate system is relatively straightforward. Taking into account that the nearest distance on the unit sphere is the angle between two locations (in radians), we define the following kernel function:

k(h)=1−hν,if h<10,otherwise, where ν is a shape parameter.

On a sphere, kernel k(h) can be represented as ks|s,r,ν=k2⋅angles,srk22⋅angles',srds', where

s - location of the kernel on sphere, r2 - radius of the kernel (in radians), angles,s=arccoss,s - the smallest angle between the two locations from the center of sphere.

This kernel generates the following random process:

Y(s)=ks'|s,r,μ⋅dWs', where Ws is white noise, EAdWs=0∀A, and varAdWs=Ads∀A.

The covariance function can be defined as: covY(s'),Y(s'')=ks|s',r,μ⋅ks|s'',r,μ⋅ds.

We assume that the covariance depends on the angle α between points only,

Cα|r,μ=ks|1,0,0,r,ν⋅ks|cosα,sinα,0,r,ν⋅ds. Note that a kernel with radius r2 generates covariance function with a range equal to r.

Unfortunately, not all kernel functions are integrable. Moreover, numerical evaluation of the positive-definite property of the covariance is problematic because it should be done with high mathematical precision. A model which overcomes some of the above-mentioned problems can be constructed based on the kernel-convolution approach proposed in Barry and Ver Hoef, 1996. Barry and Ver Hoef showed that the integral can be calculated analytically for any step kernel function. Figure 3 shows the step functions for several kernels kr,ν. The larger the number of steps the more accurate the approximation of the kernel.

Figure 3: Approximation of smooth kernel by step function with 16 steps. ν=18,14,12,1,2,4,8

Figure 4 shows a set of flexible covariance models with the range of data correlation equals π, the maximum distance between points on a sphere.

Figure 4: Covariance function with range π constructed by kernel with 64 steps

Note that the covariance shape is a function of both range and parameter ν in contrast to the covariance defined on the plane, which is a function of the νparameter only. This feature of the covariance on a sphere is important for the calculation optimization.

3 	Kriging on geoid and ellipsoid

Kriging for arbitrary (user-defined) distance metrics was developed in Krivoruchko and Gribov, 2004, using the kernel convolution approach. All calculations are performed on the grid with assigned distances between centers of the cells with a common border. Because the distance between nearby grid cells varies, the shape of the kernel depends on spatial location. A spatial process based on individually defined distances between grid cells can be constructed as follows:

- In each grid cell (i,j), model the independent random variable ξi,j with zero mean and variance σ2.

- Based on the distance value assigned to each grid cell, find the distance cdi,j,k,l to points in the specified neighborhood, where pairs (i,j) and (k,l) refer to grid rows and columns.

- Define a kernel function Kernel(cdi,j,k,l).

Assuming that all cells are equal, the spatial process is defined as Zi,j=t,sKernelcdi,j,t,s⋅ξt,st,sKernel2cdi,j,t,s with the following covariance between grid cells (i,j) and (k,l):

covZi,j,Zk,l=σ2⋅t,sKernelcdi,j,t,s⋅Kernelcdi',j',t,st,sKernel2cdi,j,t,s⋅t,sKernel2cdi',j',t,s

This kernel convolution model was partially implemented in the Geostatistical Analyst as “diffusion kernel interpolation”, which is similar to a smooth version of kriging (Gribov and Krivoruchko, 2004) with a locally varying mean value and nugget effect covariance model. Other covariance models are not currently supported because computation time is too large for commercial GIS software. However, in the case when the shape of the surface (and, correspondingly, all distances between grid cells) is known in advance, a large part of the computation can be done in advance as well for ellipsoid, or geoid, or the Earth surface defined by DEM and stored on the hard drive or the server. We discuss several cases when calculations for complex 3D bodies can be done in a reasonable amount of time (less than an hour.) We also discuss an ellipsoid approximation by a series of spheres, which allows significantly faster interpolation.

4 	3D Kriging using chordal distances

Accuracy of the predictions using kriging on a sphere and kriging with chordal distances was compared in Guinness and Fuentes, 2013. The authors of this paper developed a slow numerical version of the K-Bessel covariance for a sphere, called circular Mattern, but they did not detect any improvement of the circular Mattern over the chordal K-Bessel covariance. They concluded that “having a covariance function with flexible smoothness is more beneficial than having a covariance function that is expressed in terms of great circle distance explicitly.”

Note that the difference in the distance calculation is less than one percent for degrees smaller than 30, so that this error is typically smaller than the typical data measurement errors. Since the most important part of the semivariogram model is at small distances between pairs of points, inaccuracy in the distance calculations is acceptable in practice even for such large areas as the continental US. For example, the difference between geodesic distance between Los Angeles and New York on WGS_1984 (3939.132331 km) and chordal distance on WGS_1984 (3876.964007 km) is 62.168324 km, that is distance error is only about 1.5%.

We added an option to use chordal distance to the ArcGIS Geostatistical Analyst empirical Bayesian kriging (EBK), version 10.2.1, described in Krivoruchko and Gribov, 2014. The EBK algorithm with data transformation (described in Gribov and Krivoruchko, 2012) used to produce maps in this section is the following:

- The data are transformed to Gaussian distribution and a semivariogram model is simultaneously estimated from the data.

- Using this semivariogram, new data are unconditionally simulated and then back-transformed at each of the input data locations.

- The new data are transformed and a new semivariogram model is simultaneously estimated from the simulated data.

- Steps 2 and 3 are repeated a specified number of times. Each repetition produces a new transformation and a new semivariogram.

- Weights for the semivariograms are calculated using Bayes rule.

- Predictions and prediction standard errors are made using weights and then back-transformed taking into account a bias correction.

Advantages of the chordal distance approach include the following:

- All semivariogram models which are valid in 3D (that is almost all popular models) can be used for interpolation.

- The only modification in the EBK is a subsetting option on an ellipsoid.

- Distances between points can be calculated using any predefined shape of the Earth (or other planets), including sphere, ellipsoid (this is our default option), geoid, and topographical surfaces.

- There are no theoretical problems with the usage of the local polynomial interpolation for data detrending because it is done on the plane.

One disadvantage of the chordal distance metrics is that the interpolation using small datasets for the entire Earth is problematic. However, in practice, data with very large range of correlation are rarely available: it is hard to imagine that environmental variables near Los Angeles can be predicted using measurements made in New York.

We illustrate kriging with chordal distance using total ozone data described in Veefkind et al., 2006 (we randomly selected data on 22 February 2012.) Figure 5 shows the prediction surface in ArcGlobe and three screenshots showing the spectrum of the K-Bessel semivariogram models at three locations on the globe. The shape of the semivariograms is clearly changing from one place to another. Note that the prediction standard error of the total ozone interpolation in figure 6 is larger in the areas where the prediction is also large. This is primarily because of local Geostatistical modeling: 63,959 observations were split into subsets with 150 points each and the empirical Bayesian kriging model was fit using the data in each subset separately. Surfaces in figures 5 and 6 were created in less than two minutes on a standard laptop computer.

Figure 5: Total ozone on February 22, 2012, in DU (Dobson unit.) One Dobson unit is 
equivalent to a layer of pure ozone 0.01 mm thick at standard temperature and pressure

Figure 6: Prediction standard error of the total ozone on February 22, 2012, in DU (Dobson unit)

Acknowledgement

We thank to Witold Fraczek, Esri, for help with figures 1 and 2 creation.

References

[1] Barry, R. P. and Ver Hoef, J. M. (1996), Blackbox kriging: spatial prediction without specifying variogram models, Journal of Agricultural, Biological, and Environmental Statistics, Volume 1, Number 3, pp. 297-322.

[2] Gribov, A., and Krivoruchko, K. (2004), Geostatistical Mapping with Continuous Moving Neighborhood, Mathematical Geology, Volume 36, Number 2, pp. 267-281.

[3] Gribov A. and Krivoruchko K. (2011), Local Polynomials for Data Detrending and Interpolation in the Presence of Barriers, Stochastic Environmental Research and Risk Assessment, Volume 25, Issue 8, pp. 1057-1063.

[4] Gribov A. and Krivoruchko K. (2012), New Flexible Non-parametric Data Transformation for Trans-Gaussian Kriging, Geostatistics Oslo 2012, Quantitative Geology and Geostatistics, Volume 17, Part 1, pp. 51-65, Springer Netherlands.

[5] Guinness J. and Fuentes M. (2013), Covariance functions for mean square differentiable processes on spheres. Under review.

[6] Hitczenko, M. and Stein, M. L. (2012), Some theory for anisotropic processes on the sphere, Statistical Methodology, 9, 211-227.

[7] Krivoruchko K. (2012), a) Empirical Bayesian Kriging and b) Modeling Contamination Using Empirical Bayesian Kriging, ArcUser.

http://www.esri.com/news/arcuser/1012/fall2012.html

[8] Krivoruchko, K. and Gribov, A. (2004), Geostatistical interpolation in the presence of barriers, geoENV IV - Geostatistics for Environmental Applications, Quantitative Geology and Geostatistics, Volume 13, pp. 331-342, Springer Netherlands.

[9] Krivoruchko K. and Gribov A. (2014), Pragmatic Bayesian kriging for non-stationary and moderately non-Gaussian data, Mathematics of Planet Earth, Lecture Notes in Earth System Sciences, Springer, pp. 61-64.

[10] Veefkind, J.P., J.F. de Haan, E.J. Brinksma, M. Kroon, and P.F. Levelt (2006), Total ozone from the ozone monitoring instrument (OMI) using the DOAS technique, IEEE Transactions on Geoscience and Remote Sensing, Volume 44, Number 5, pp. 1239-1244.