Variogram-based Template Selection for Multiple-point Geostatistics

Xue Li1, Tao Huang2, Wei Peng3, DeTang Lu4

Department of Modern Mechanics, University of Science and Technology of China

1 snowlee@mail.ustc.edu.cn

2 thwang@ustc.edu.cn

3 wpengme@ustc.edu.cn

4 dtlu@ustc.edu.cn

Abstract. In multiple-point geostatistics, patterns extracted by scanning the training image (TI) with a spatial template are essential for the spatial phenomena under study. The size and geometry of a suitable template have a significant impact on the identification of the most relevant geological features and the realization rationality of computing and storage. The necessary trial-and-error process in practice for the customizable templates is cumbersome and subjective. In this paper, we attempt to determine a robust template which is restricted within a most influential scope based on the spatial correlation of TI analyzed from variogram model. A quantitative measurement information entropy theory as well as several multiple-point simulation results is posed to discuss the effectiveness of our method.

Keywords. Template selection; multiple-point geostatistics; variogram; information entropy

1 	Introduction

The scanning template used to extract spatial features of TIs has significant impact on multiple-point simulation implementations [1]. Though it is necessary in multiple-point algorithms, the template is usually defined by the user without properly selection guide. In this paper the optimized template is defined considering the heterogeneity from an omni-direction variogram model. The effects are discussed with information entropy theory and multiple-point simulation results.

In general, to obtain more information more points should be contained in a scanning template, which in turn calls for larger memory space and heavier computational burdens [2, 3]. Therefore, template size should be carefully selected to get a balance, which usually needs several attempts to find well fits in practice. An alternative way is to change the geometry of a scanning template. Beyond the widely used isotropic templates, the growth of features can be observed from conditional data with a special template representing the dominant pattern [4]. But the utilization of experimental covariance in this algorithm may introduce extreme values. In this paper an attempt is made to investigate the influence of geological heterogeneity on the template selection by using the parameters of orientation variogram models. Then several experiments are given on different datasets. The improvements are compared with a criterion known as information entropy theory [5, 6] and the observation of several simulation results. Finally, the prospects of template selection are discussed for further improvement.

2 	Scanning template and data event

In multiple-point statistics, the TI is scanned by a data template to find replicates termed as data events. A n-point data template τn comprises a central location u and n locations uα= u +hα (α = 1,2,…,n) where hα is the vector defining the template geometry. For an attribute S taking K possible states {sk; k =1,2,…,K}, a data event dn of size n centered at location u is constituted by the n template vectors and the n data values which is defined as [7]:

dn={S(uα)=; α=1,2,…,n}.                            (1)

where S(uα) states the location of uα within the template.

The probabilities of occurrences of the data events dn are denoted as:

p(dn)=Prob{S() =; α=1,2,…,n},                       (2)

and the probabilities of occurrences of the data event dn with a certain center value sk is:

p(sk, dn)= Prob{S(u)=sk and S() =; α=1,2,…,n }.             (3)

The K possible outcomes of the random variable S(u) are characterized by their conditional probability distribution function (cpdf) defined as:

.                                (4)

In the process of scanning a TI with a given template, a replicate is obtained when a data event in the TI has the same geometric configuration and the same data values as dn associated with τn and the replicate number c(dn) is obtained. Similarly, the number of replicates of data event dn with the center value sk is denoted as ck(dn). Thus the conditional probability can be calculated as:

.                                (5)

The fundamental principle of multiple-point stochastic simulation is conditioning the cpdfs inferred from a TI and copying the corresponding pattern to the model to be simulated. Consequently the data events recorded should be elaborate enough to reflect the features of a ti on the other hand they couldn’t be too specific to retain the essence. This paper focuses on the effort to define a suitable scanning template which can optimally represent the dominant pattern of the TI and improve simulation results.

3 	Quantitative criteria

In order to choose a proper template, some criteria of optimality need to be built. Recently the concept of information entropy was introduced to quantify the amount of information extracted from a TI which has already been used in template size selection [5] for rectangular template. Similar to the treatment of English language [8], in this template size selection algorithm, entropy is used as a statistical measure to take patterns as realizations of random variables. The entropy of a pattern can be computed as:

,…………………    ……………….. (6)

where K is the number of possible outcomes of the random variable and pi represents the probability mass function. According to information theory, the template with a larger entropy containts more information. With the increase of template size, the entropy curve rises rapidly at first as the amount of information needed to compress the patterns is increasing. Then the growth space slows down implying that the template size has covered stationary features of the TI. As a consequence, an optimal template size would be determined at the inflection point by the entropy criterion [5].

In terms of geometric configuration of a search template, the optimal window represennting the dominant training features could also be figured out with entropy values. With the same size, the optimal template could capture a data event set that contains smaller kinds of data events but more replicates of each pattern. This characteristic leads to a probability distribution that shows concentration of the essential features rather than uniform occurances of all possible patterns. Thus the geological features that appear most could be easy to reproduce in multiple-point simulation. In order to measure this tendency of patterns, the previous entropy function is deformed.

3.1 	Deformation

Instead of taking every point of the pattern as a realization, the replicate of each pattern is concerned to measure the information captured by a template form a TI. Therefore the entropy of a TI is calculated by the probability distribution function of p(Sk, dn) defined as:

,                      (7)

where N represents the number of occurrences of the joint data events.

This entropy is a measurement of the average uncertainty of information extracted from a priori TI. According to the relationship of information entropy and probability distribution [9], which shows a positive correlation between the uniformity of a probability distribution and the entropy value, a smaller value is expected conversely as it means a greater probability of occurrences of essential patterns.

3.2 	Illustration

For instance, a random scattered strip-type binary TI is scanned using tree kinds of four-point templates as shown in Figure. 1.

Figure 1: A random strip-type binary TI and three four-point scanning templates

The white and gray grids of the original image are numbered as 0 and 1 respectively. Theoretically, 24=16 kinds of patterns are generable, but only parts of them occur. The probability distribution of patterns is listed in Table 1. Because of the same horizontal stripe type with the TI, template (c) identifies the essential features and presents a most skewed probability distribution in the meantime. In contrast, the vertical patterns captured by Template (d) show a more uniform distribution and a highest entropy value. Consequently, entropy could be utilized to estimate the concentration of dominant features with a relevant value, namely, Template (c) is optimal.

Table 1: Probability distributions of patterns obtained by each template and the relevant information entropies

| dn | Template (b) |  | Template (c) |  | Template (d) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | c(dn) | p(dn) | c(dn) | p(dn) | c(dn) | p(dn) |
| 0000 | 7 | 14.3% | 20 | 50.0% | 0 | 0% |
| 0001 | 0 | 0% | 0 | 0% | 0 | 0% |
| 0010 | 0 | 0% | 0 | 0% | 0 | 0% |
| 0011 | 14 | 28.6% | 0 | 0% | 8 | 20% |
| 0100 | 0 | 0% | 0 | 0% | 0 | 0% |
| 0101 | 0 | 0% | 0 | 0% | 8 | 20% |
| 0110 | 0 | 0% | 0 | 0% | 0 | 0% |
| 0111 | 0 | 0% | 0 | 0% | 0 | 0% |
| 1000 | 0 | 0% | 0 | 0% | 0 | 0% |
| 1001 | 0 | 0% | 0 | 0% | 8 | 20% |
| 1010 | 0 | 0% | 0 | 0% | 8 | 20% |
| 1011 | 0 | 0% | 0 | 0% | 0 | 0% |
| 1100 | 21 | 42.8% | 0 | 0% | 8 | 20% |
| 1101 | 0 | 0% | 0 | 0% | 0 | 0% |
| 1110 | 0 | 0% | 0 | 0% | 0 | 0% |
| 1111 | 7 | 14.3% | 20 | 50.0% | 0 | 0% |
| Total | 49 | 100% | 40 | 100% | 40 | 100% |
| Entropy | 1.84 |  | 1 |  | 2.32 |  |

4 	Template optimization based on variogram models

Considering the heterogeneity, neighborhood selection based on spatial correlation is conventional in geostatistical fields such as kriging methods [10, 11]. For multiple-point simulation, a template optimization algorithm was proposed [4]. By calculating covariance values between the central node of the study grid and all the other nodes, the corresponding locations of the n top values are chosen to form the n-point template. This method can increase the number of relevant data events found, however, it is risky as it may be sorely influenced by even a single point.

Following the previous work, we attempt to improve the template selection algorithm based on variogram models which can remove the extreme values. The experimental variogram can be calculated as:

.                    (8)

An omni-direction variogram calculates on at least 4-direction vectors for 2D TI and 16-direction vectors for 3D cases. By fitting all directional variograms simultaneously, the parameterized model is robust to interpret spatial anisotropy [12]. This proposed method defines the optimized scanning template by delivering the anisotropy obtained from the variogram model to the geometrical configuration of the search neighborhood. A widely used neighborhood definition algorithm in multiple-point simulation is to specify a 3D ellipsoid as a search volume, which is designated by three radii and three angles [13]. Figure. 2 illustrates a 2D ellipsoid cross section sample on XY plane. In this paper, the same algorithm is applied with the six parameters coherent to the spatial anisotropy of the TI obtained by the variogram functions.

Figure 2: The XY plane cross-section of a search ellipsoid

5 	Experiments

To investigate the relationship among spatial anisotropy, template configuration and extracted features, several TI with similar features are used and shown in Figure 3:

(a) a 2D artificial TI with scattered ellipse with radii 5 and 13 in X and Y direction;

(b) a 3D TI with user-defined shapes provided by SGeMS [13].

(a)			  (b)

Figure 3: Two training images

As discussed above, the spatial heterogeneity of the TIs are analyzed at first. For TI (a), Figure 4 (a) exhibits the experimental variograms calculated on all directions with Eq. (8), and the ranges obtained from Gaussian modeling are displayed in a rose histogram Figure 4 (b) which is an intuitional way to express variance ellipse. According to Figure 4 (b) an optimized search ellipse with radii ratio1:3 in X and Y direction contains the most relevant nodes to the central node. As a contrast, an isotropic template and a template obtained from a ninety-degree rotation are also used to evaluate the sensitivity of template selection.

(a)

(b)

Figure 4: Variogram analysis of TI (a): (a) experimental variograms on four directions; (b) rose histogram of Gaussian modeling

The same experiment on TI (b) shows a pure nugget effect on vertical direction while the anisotropy on XY plane shows a 2D ellipse with a ratio of 4:3 for radii on X and Y axes. The relative parameters evaluated from the exponential variogram models are displayed in Figure 6 (b). Similarly three templates, i.e. homogeneous spherical structure, circular structure on XY plane and ellipse structure with the radius of 4:3 on X and Y axes, are used in experiments.

(a)

(b)

Figure 5: Variogram analysis of TI (b): (a) experimental variograms on five directions; (b) rose histogram of Exponential modeling on XY plane

Then using the spatial configurations defined prevsiously and fixed template size 60, entropies computed by Eq. (7) are shown in Table. 2. The relatively low entropy values show that more data events describing essential features were captured by the optimized template than by the arbitrary templates on both tests.

Table 2: Entropies of data events obtained by different scanning templates

| TI (a) |  |  |  |
| --- | --- | --- | --- |
| Ratio of search template (X:Y) | 3:1 | 1:1 | 1:3 (optimized) |
| Entropy | 6.34 | 5.62 | 5.17 |
| TI (b) |  |  |  |
| Ratio of search template (X:Y:Z) | 1:1:1 | 1:1:0 | 4:3:0 (optimized) |
| Entropy | 10.60 | 6.41 | 6.36 |

Furthermore, several simulations are also implemented with SNESIM algorithm with the same templates set up in Table 2 on both TIs. As exhibited in Figure 6, significant improvements of simulation results certify the effectiveness of the optimized template. Considering Table 2 at the same time, there is a correlation between better simulation results and lower entropy values.

(a)                      (b)                        (c)

(d)                    (e)                     (f)

Figure 6: Simulation results with different scanning templates: (a)-(c) for TI (a);

(d)-(f) for TI (b)

6 	Conclusion and Discussion

The scanning template has an important impact on both simulation results and computational burdens of multiple-point geostatistical simulations. This paper proposes a template selection approach based on the variogram analysis which robustly limits a most influential search region of the TI. The modified information entropy calculated with the distribution of joint data events is also presented as a criterion which indicates more essential patterns extracted at a lower value. Simulation results on different data sets with similar features show significant improvement using the optimized template selected at the lowest entropy point.

The effective improvements show the potential for further researches on more complex TI s that other factors may influence at the same time. For instance, a continuous channel image has a longer half range of its variogram model beyond the coverage of a search template. It is supposed that quantified study could reduce uncertainty of geostatistical simulations.

References

[1] Guardiano, F. B. and R. M. Srivastava (1993). Multivariate geostatistics: beyond bivariate moments. Geostatistics Troia’92, Springer: 133-144.

[2] Daly, C. and J. Caers (2010). Multi-point geostatistics–an introductory overview, First Break 28(9).

[3] Straubhaar, J., P. Renard, et al. (2011), An Improved Parallel Multiple-point Algorithm Using a List Approach, MATHEMATICAL GEOSCIENCES 43(3): 305-328.

[4] Barrera, A., J. Ni, et al. (2005), Multiple Point Geostatistics: Optimal Template Selection and Implementation in Multi-Threaded Computational Environments, Geostatistics Banff 2004, Springer Netherlands, 14: 621-630.

[5] Honarkhah, M. and J. Caers (2010), Stochastic simulation of patterns using distance-based pattern modeling, MATHEMATICAL GEOSCIENCES 42(5): 487-517.

[6] MacKay, D. J. (2003), Information theory, inference and learning algorithms, Cambridge university press.

[7] Strebelle, S. (2002), Conditional Simulation of Complex Geological Structures Using Multiple-Point Statistics, Mathematical Geology, 34(1): 1-21.

[8] Shannon, C. E. (1948), A mathematical theory of communication, Bell System Technical Journal, 27(1): 379-423.

[9] RRNYI, A. (1961), On measures of entropy and information, Fourth Berkeley Symposium on Mathematical Statistics and Probability.

[10] Rivoirard, J. (1987), Two key parameters when choosing the kriging neighborhood, Mathematical Geology, 19(8): 851-856

[11] Vann, J., S. Jackson, et al. (2003), Quantitative kriging neighbourhood analysis for the mining geologist-a description of the method with worked case examples, Proceedings Fifth International Mining Geology Conference.

[12] Gringarten, E. and C. V. Deutsch (2001), Teacher's aide variogram interpretation and modeling, Mathematical Geology 33(4): 507-534.

[13] Remy, N., A. Boucher, et al. (2009), Applied Geostatistics with SGeMS - A User's Guide, Cambridge University Press, New York 279.